BRIEFING

LCP BREF: Definition and application of peak and emergency mode

In the EIPPCB’s background paper, it is stated, “The possible impact of the load mode was assessed for each BAT conclusion and BAT-AEPL individually, based on available techno-economic information and looking at implemented techniques and achieved performances.“  Following this assessment, the EIPPCB proposed to: “Assess case-by-case, for each plant category, the need to address the influence of load modes in the fuel-specific BAT conclusions.”

The EIPPCB proposes to apply the following definitions for different load modes in the new BAT Conclusions which will take effect from 2020:

Table 1

A number of exemptions from compliance with BAT-AEPLs are proposed for different categories of power plant and industrial process installations when operating in what are dubiously defined as peak and emergency mode (see Annex 1). Derogations from compliance with BAT-AEPLs for peak mode and emergency mode operation are proposed based on ‘techno-economic’ information but the detailed reasoning is not set out in the background paper. Whatever the reasoning, we set out below why such derogations should not apply to power generation operating in the liberalised wholesale electricity and balancing markets. Any derogation should be limited to out-of-market emergency conditions.

- A mode defined as operating for less than 500 hours extends in the vast majority of cases to plants that are in fact competing in the energy market in peak mode, not emergency mode. Emergency conditions refer to situations where the system is seriously disturbed, is operating outside normal parameters and where there is a real risk of a full or partial shutdown. Emergency conditions therefore apply when the wholesale energy and balancing markets (operating reserves) have not been able to clear, the gate has closed and the system operator uses emergency reserves to balance the grid. These resources are therefore out of market, though competitively procured.

Emergency conditions do not refer to scarcity events or normal standards of supply reliability, which in Europe are typically set at around 3 - 8 hrs of unsupplied energy per annum. Strategic or other reserve plant would run to maintain these standards and use of such reserves can be expected to increase with growth in the share of RES in the power mix and with EU market design evolving to improve functioning of the wholesale electricity markets.

One referent is the process followed in New England, which was developed through cooperation with and collaboration among that region’s environmental and energy regulators, and the grid operator.  This process resulted in a tightly defined and agreed up definition of “emergency” and when such units could operate.

In addition, a recent court ruling in the US provides evidence that defining emergency mode according to a minimum number of hours of operation is a flawed approach. The U. S. Court of Appeals, District of Columbia Circuit, ruled that the EPA’s proposed regulations to increase emergency mode operation from 15 hours to 100 hours for reciprocating internal combustion engines (also known as the “RICE” rule) could not be justified. The EPA must now rewrite that rule, and all expect a more stringent definition of “emergency” to be instituted.

- At times of peak energy demand when RES is not available, air quality is more likely to be poor. The derogation, as framed, allows outcomes that the Industrial Emissions Directive is trying to avoid.

Plant owners will rationally operate their plant when prices are high and that is likely to be when RES is not available and energy demand is high. At such a moment, thermal generation will typically be dominant in the power mix. Furthermore, as the share of wind and solar generation in power mix increases, then scarcity events could become more common and these can occur during anti-cyclonic events that are associated with poor air quality and can last for a couple of weeks. Conversely, "emergency states" will not generally be associated with anti-cyclonic conditions and, in fact, are more likely to be associated with cyclonic systems when air quality is not such an issue. This is another reason why basing derogations on expected hours of running is arguably the wrong approach.

U.S. EPA emission standards are distinguished by the capacity of the boiler or electric generating unit and the date that construction commenced on the affected unit, and not by the per capita income of a state or the number of hours of operation.  States with unhealthy air quality (ozone and fine particulate matter- PM2.5) have more stringent emission standards and restrict the number of hours that a generator may operate in an emergency mode.  An affected unit can receive an exemption from an EPA or state “best standard” if and only if the owner or operator takes a permit restriction that limits emissions to a threshold below that of a major source (limited to emissions of no more than 10-100 tons per year, depending on the severity of air quality) and accepts both short and long-term emission standards to assure compliance with the permit restriction.

The graph (Figure 1) below illustrates how high NOX emissions (largely from peaking plants) correlate with high energy demand for the case of Connecticut (detailed study completed by Connecticut Department of Environmental Protection).

Figure 1

Various peak reduction studies how significant emissions are at times of peak electricity demand. For example: an ISO-NE analysis of regional data from 2005-2008 reveals that a 500 MW of peak reduction equates to around 3.6 tonsNOx/100MW; integrated resource planning modeling for 2013 (using 2009 Celt Forecast) concluded that a 300 MW of peak reduction would equate to around 4.2 tons NOx/100MW); The Connecticut Department of Environmental Protection (DEP) analysis of top 4 demand days for the period 2005-2009 showed that 600 MW of peak reduction equated to around 4.4 tons NOx/100MW.

- Operation under peak or emergency mode according to the proposed definition will highly likely occur in the competitive wholesale electricity (energy and balancing) markets where a variety of low emission alternatives exist. Allowing a sub-section of energy resources to benefit from air quality regulation derogations in a competitive marketplace for any length of time is market distorting.

- The Internal Energy Market legislation (including the Third Package and Network Codes) establishes a liberalised and integrated energy market. Economic dispatch applies and is in accordance with a merit order where the lowest cost generation is dispatched to meet energy demand at any point in time. Power clearing prices are highest when net energy demand (not served by renewable energy generation) peaks. Plant exempt from the BREF’s BAT-AELs according to the proposed peak and emergency mode definitions will operate when it is most profitable to do so.

- With the possible exception of generation literally restricted to operating during actual emergencies, declared by the system operator under specified conditions, there is simply no good reason to allow thermal generation plant bidding into the market to run unabated, without internalizing external costs, enabling it to undercut the bid prices of less polluting alternatives, pushing these competing alternatives out of market.

- Reliability (system adequacy) cannot be a reason for peak mode derogation; power sector market design and power sector policies should determine wholesale market outcomes and the need to intervene to ensure reliability. The design of BAT-AEPLs should not be designed around power market functioning, perceived or real; instead, the BAT-AEPLs should effectively determine whether or not a combustion plant is eligible to participate in the power market.

- Massive investments have been made in low emission energy technologies in response to market signals, policies and stated long term goals.  As a result there is an ample surplus of generating capacity in most markets, so that there is no generation adequacy rationale for the proposed exemptions. It is the regulator’s role to make sure that all options able to provide services required by the market can do so and are treated fairly.

- Recently, there have been cases of new, efficient, relatively low emission gas plants being mothballed while older, more polluting mid-merit plant continues to operate profitably. The low Emissions Trading Scheme (ETS) allowance price has not been able to bridge the difference between the clean dark spread and clean spark spread. Derogations from compliance with BAT-AEPLs, as proposed in this BREF, will only exacerbate this situation. There is thus a high risk in Europe today, and already a reality in some regions, that investment will flow to the most polluting power generation assets, rendering less polluting more flexible generation uncompetitive. This costs EU citizens dearly in terms of worse air pollution and climate change than is necessary. This is a key reason why there should be a level playing field as far as possible with respect to setting of the BAT-AEPLs for power generation from different fuels and for new and existing plants. In wholesale electricity markets, and capacity markets where they exist, no such distinctions are applied. The design of the EU ETS is also free of such distinctions. Non-discrimination in markets is, rightly, a principle strongly promoted by industry.

- Some plant owners may be concerned that price caps in power markets prevent scarcity pricing and that average wholesale power market prices are at present relatively low. Overcapacity in most European power markets is a key reason for the latter. With EU policies continuing to promote RES and energy efficiency, excess marginal capacity can be expected to retire. Once excess capacity is retired from currently oversupplied power markets, power prices will rise. At the same time, regulators will rightly be concerned about ensuring system adequacy with growing shares of RES in the power mix as the system will need to be supplied and in balance at times of low RES availability. This issue, however, needs to be dealt with through power market design reform and power sector policies. EU-wide reforms are indeed underway to ensure power market design is fit for purpose for an EU power mix with a high share of RES. The Energy Union Communication, recently adopted by Council, reinforces political commitment to completing the internal energy market and to accelerating this process with introduction of a new legislative proposal on market design, including reserves.

- There are no compelling technical reasons that abatement is not possible for certain hours of operation.

See http://www.iso-ne.com/static-assets/documents/committees/comm_wkgrps/prtcpnts_comm/eag/mtrls/2013/apr122013/epa_rteg_meeting_3_18_2013.pdf

The Independent System Operator for New England (ISO-NE) has defined stages of what actions will trigger a voltage reduction or requirements to call upon emergency generators to commence operation. http://www.iso-ne.com/static-assets/documents/rules_proceds/operating/isone/op4/op4_rto_final.pdf

United States Court of Appeals, District of Columbia Circuit (this is the central court for hearing of federal issues in the United States). Link to decision: http://www.rtoinsider.com/wp-content/uploads/DC-Circuit-ruling-on-backup-gens-DR-5-1-15.pdf

U.S. EPA New Source Performance Standards for fossil-fuel fired steam generators, subparts D, Da, Db and Dc. Complete list of all NSPS requirements found at: http://www.ecfr.gov/cgi-bin/text-idx?tpl=/ecfrbrowse/Title40/40cfr60_main_02.tpl

The number of hours permitted for a “emergency engine” to operate includes those used for periodic testing, typically one-half hour every fortnight, or about 25 hours a year.

As a practical matter, the emission limits for such “minor sources” is equivalent or more stringent than those of “best technologies”. Restricting emissions avoids the need to pay annual fees for each ton of emissions and avoids certain record keeping and reporting requirements.

Weblinks for the three studies or to Chris’s powerpoint

See European Commission website: https://ec.europa.eu/energy/en/topics/markets-and-consumers/market-legislation

See ENTSO-E website: http://networkcodes.entsoe.eu/

Indeed the EU ETS is to be reformed, but as a quantity based instrument and with political reluctance to set an ambitious cap that will result in higher prices, not forgetting the volatility of wholesale coal and gas prices, it will likely never be able to guarantee a merit order correlated to CO2 performance.

See: http://www.eurelectric.org/news/2013/eurelectric-welcomes-ec%E2%80%99s-pro-market-stance-on-public-intervention/

See http://ec.europa.eu/clima/policies/2030/index_en.htm for summary of policies and legislation.

European Commission, “Energy Union Package”, COM(2015)80 final, 25.02.2015, Brussels.