- 5.8.1.3.y Capture and control of POPs emissions from physico-chemical treatment of POPs waste

a)	Control of emissions to air from negative air system (NAS)

- The NAS gas outlets contain filters (using most appropriate method in order to prevent POPs releases and/or formation), and verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene).

b)	System either works in reducing atmosphere or uses control of emissions to air from processing units including physico-chemical treatment reactors (for techniques not working in reducing atmosphere),

- Air Pollution Control System (APS) is applied using most appropriate technique to control POPs releases (most often activated carbon). Unitentional POPs formation has to be prevented to maximum extend possible during operation of the whole technology unit including APS. Verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene).

- APS is also applied to indirect thermal desorption kiln (when applied) and/or any additional units (e.g.

furnace for metal decontamination). APS can be either same for emission control from physico-chemical technology reactor or separate one.

- See above.

Monitoring of air emissions

- Monitoring of all point outlets to air:

Semi-continual monitoring (3x8 hours sampling every month) of POPs is carried out and includes all POPs contained in material (waste) treated by technology as well as it monitors PCDD/Fs, DL PCBs (expressed in TEQ)  and hexachlorobenzene

Table 5.xxIII: BAT-associated emission levels (BAT-AELs) for POPs emissions to air from physico-chemical treatment of POPs waste – based on performance of technologies listed above in subchapter xxxx (see also Table 5.xx2).

| BAT-associated emission levels (BAT-AELs) for PCDD/Fs and/or PCDD/Fs and dioxin-like PCBs to air physico-chemical treatment of POPs contaminated waste | Unit | BAT-AEL (Average of samples obtained during one control measurement) |
| --- | --- | --- |
| PCDD/Fs | ng I-TEQ/Nm3 | 0.05 |
| PCDD/Fs + DL PCBs | ng I-TEQ/Nm3 | 0.05 |
| BAT-associated emission levels (BAT-AELs) for PCDD/Fs and/or PCDD/Fs and dioxin-like PCBs to air physico-chemical treatment of POPs contaminated waste | Unit | BAT-AEL (Emission factor set as average of one year operation) |
| PCDD/Fs | µg (TEQ)/ton of treated waste | 0.02 |

Achieved environmental benefits

All techniques aim at increasing the environmental performance of destruction and irreversible transformation of POPs in waste and to control POPs emissions during this process.

Environmental performance and operational data

Cross-media effects

 Possible consumption of reagents for the decontamination and/or the abatement system.

 Potential residues containing POPs from APS and/or NAC.

Technical considerations relevant for applicability

Generally applicable.

Economics

No information available.

Driving force for implementation

 POPs regulation (EC 850/2004).

 Stockholm Convention

Example plants

Brief description of the technology used for decontamination of buildings and soil contaminated with PCDD/Fs and OCPs

References: