New suggested BAT for section 6.4.7: BAT conclusions for the decontamination of equipment and waste containing POPs

BAT xa. In order to improve the general environmental performance of POPs (listed under Stockholm Convention) destruction technologies, BAT is to apply all of the techniques given below:

| Technique |  | Description |  |
| --- | --- | --- | --- |
| a | Design measures to prevent dispersion of POPs from the storage and treatment areas |  |  Dedicated storm and run-off water collection system.  Resin coating applied to the whole concrete floor of the storage and treatment areas without using POPs in that resin itself. |
| b | Decontamination of soil and other solid wastes which can create dust dispersion |  | Mechanical treatment of solid wastes (including soil, debries etc.) contaminated with POPs (materials subjected to decontamination) and the processing units are covered with a "tent" with Negative Air System (NAS). |
| c | Implementation of staff access rules to prevent dispersion of contamination |  |  Accesses to storage and treatment areas are locked.  Special qualification is required to access the area where the waste or polluted equipment is handled.  Separate 'clean' and 'dirty' cloakrooms to put on/take off individual protective outfit. |
| d | Control of emissions to air from NAS |  | The NAS gas outlets contain filters (using most appropriate method in order to prevent POPs releases and/or formation), and verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene). |
| e | Management of waste treatment residues |  |  Residues are treated in physico-chemical treatment technologies for POPs destruction. |
| f | Management of the proces with regards to unintentional POPs formation |  | Technology runs within conditions which do not lead to unintentional POPs (e.g. PCDD/Fs) formation when they are described for certain technology |
| g | Management of treated waste (soil in particular) when it doesn’t reach the target value of POPs contaminant |  | Treated material is re-processed in technology when it doesn’t meet required target concentration of POPs set in permit which should not be higher than standards set for the purpose of material use (e.g. standards set for industrial areas national legislation etc.) |
| h | Monitoring of outlets |  | All outlets are controlled for their content of POP contaminant(s) treated in the waste (e.g. DDT and other relevant OCPs in case of contaminated material from obsolete pesticides storage), as well as for unintentional POPs content (PCDD/Fs as minimum requirement). These measurements should be carried out in order to evaluate destruction efficiency (DE) and destruction and removal efficience (DRE) of applied technology |

[This BAT conclusion is based on information given in Section xxxxx]

Table 6.x1: BAT-associated destruction efficiencies (DE) for physico-chemical treatment of POPs waste

| BAT-associated destruction efficiency levels for POPs for physico-chemical treatment technologies of POPs contaminated waste | Unit | BAT-DE (as minimum level) (DE calculation is based on measurement of all inlet and outlet streams) |
| --- | --- | --- |
| PCDD/Fs | % | 99.999 |
| OCPs (DDT, HCB, lindane and other listed in Annexes to Stockholm Convention) | % | 99.999 |
| PCBs | % | 99.999 |
| Chlorobenzenes | % | 99.999 |

6.4.7.2 Emissions to air

BAT xb. When POPs contaminated wastes (including contaminated soil) are treated in physico-chemical treatment following air emission controls are applied, BAT is to use combination of the techniques given below.

| a | Control of emissions to air from negative air systém (NAS) | The NAS gas outlets contain filters (using most appropriate method in order to prevent POPs releases and/or formation), and verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene). |
| --- | --- | --- |
| b | System either works in reducing atmosphere or uses control of emissions to air from processing units including physico-chemical treatment reactors (for techniques not working in reducing atmosphere), | Air Pollution Control System (APS) is applied using most appropriate technique to control POPs releases (most often activated carbon). Unitentional POPs formation has to be prevented to maximum extend possible during operation of the whole technology unit including APS. Verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene). |
| c | APS is also applied to indirect thermal desorption kiln (when applied) and/or any additional units (e.g. furnace for metal decontamination). APS can be either same for emission control from physico-chemical technology reactor or separate one. | See above. |

BAT xc. When POPs contaminated wastes (including contaminated soil) are treated in physico-chemical treatment following air emissions monitoring is applied.

| a | Monitoring of all point outlets to air | Semi-continual monitoring (3x8 hours sampling every month) of POPs is carried out and includes all POPs contained in material (waste) treated by technology as well as it monitors PCDD/Fs, DL PCBs (expressed in TEQ) and hexachlorobenzene |
| --- | --- | --- |

Table 6.x2: BAT-associated emission levels (BAT-AELs) for POPs emissions to air from physico-chemical treatment of POPs waste

| BAT-associated emission levels (BAT-AELs) for PCDD/Fs and/or PCDD/Fs and dioxin-like PCBs to air physico-chemical treatment of POPs contaminated waste | Unit | BAT-AEL (Average of samples obtained during one control measurement) |
| --- | --- | --- |
| PCDD/Fs | ng I-TEQ/Nm3 | 0.05 |
| PCDD/Fs + DL PCBs | ng I-TEQ/Nm3 | 0.05 |
| BAT-associated emission levels (BAT-AELs) for PCDD/Fs and/or PCDD/Fs and dioxin-like PCBs to air physico-chemical treatment of POPs contaminated waste | Unit | BAT-AEL (Emission factor set as average of one year operation) |
| PCDD/Fs | µg (TEQ)/ton of treated waste | 0.02 |