Mini - BREF

treatment in shredders of fridges, freezers and air-conditioning units

Controversial points:

- Scope of this document

Should the scope be limited to temperature exchange equipment that contains Ozone Depleting Substances and Hydrofluorocarbon (i.e. that there is no legal requirement for temperature exchange equipment in which gases that are ozone depleting or have a global warming potential (GWP) above 15  have been substituted, and that the substitutes have a GWP <15  to be shredded in the specialist shredders used to capture ODSs)

or

The scope should include all temperature exchange equipment, which contains CFC, HCFC, HFC or HC 2

- Legal requirements

Should the document state legal requirements in strict accordance with Directive 2012/19/EU on waste electrical and electronic equipment (WEEE)

Or

Should the document include requirements found in the voluntary CENELEC Standards that go beyond the legal requirements of Directive 2012/19/EU on waste electrical and electronic equipment (WEEE)

- Treatment of gases

Proposal re destruction on site: Removed VFCs, VHCs and mixtures thereof shall either be transported to an authorised facility for recovery or disposal, or recovered or destroyed by onsite conversion.[19] or

Alternative proposal : Ozone-depleting gases must be treated in accordance with Regulation (EC) No 1005/1009 (Source: Directive 2012/19/ EU Annex VII No 2).]

It is also for these reasons, see Controversial point 1, that only the appliances containing ozone-depleting substances (ODS) or hydrofluorocarbon have to be processed using the techniques described in this document. Therefore, the equipment which are not containing ODS and HFC don’t fall within the scope of this Document.

- Quoted limit values

Data collected from the TWG Questionnaires should be used to inform the replacement of respective values in this document

June 2014

This page is left blank intentionally

Contents

Figures and tables	7

Chapter 1 – General information	9

1.1 – The purpose of refrigerator recycling facilities	9

1.2 – The types of shredders in refrigerator recycling plants	10

1.2.1 Twin – Shaft shredder	11

1.2.2 Four-shaft shredder	11

1.2.3 Super chopper	13

1.2.4 Rotorshredder	14

1.3 – General information for refrigeration devices	16

1.3.1. General information for fridges	16

1.3.2 General information for air condition	17

1.3.3 Refrigerants and blowing agents	19

1.4 The process	20

1.5 – Refrigerator recycling plants in Europe	21

1.6 - Ecological and economic aspects of the refrigerator recycling process	22

1.7 - Economic and institutional aspects of the refrigerator recycling process	23

1.8 – General environmental issues related to shredders	23

1.8.1 – Waste	23

1.8.2 - Use of energy	24

1.8.3 - Contamination of soil and ground water	24

1.8.4 – Pollution of air	24

Chapter 2 – Applied processes and techniques	25

2.1 – Common techniques applied to the sector	25

2.1.1 – Delivery and acceptance	25

2.1.2 – Pre-treatment	26

2.1.3 - Stage 1:  Extraction of the refrigerants and refrigerator oil	26

2.1.4 - Stage 2: treatment of the drained refrigerators	28

2.2 – Emission reduction technologies at refrigerator recycling plant	32

2.2.1 – Technical measures for degasification inside the shredding area	32

2.2.2 – Technical measures for the reduction of dust in exhaust air	32

2.2.3 – Technical measures for the reduction of gaseous organic compound in exhaust air	33

2.2.4 – Technical measures for the reduction of noise	33

2.2.5 – Technical measures for water treatment	34

Chapter 3 – Current consumption and emission levels	35

3.1 Energy input and auxiliary materials	35

3.2 – Water Emissions	36

3.3 – Air emissions	36

3.3.1 - Dust	37

3.3.2 ODS	37

3.3.3 – PCB	37

3.3.4 - Mercury	37

Chapter 4 – Techniques to consider in the determination of BAT	39

4.1 Delivery and acceptance	39

4.1.1 Transportation and storage	39

4.1.2 Sorting	39

4.2 Technologies for the reduction of air emissions	41

4.2.1. Techniques to eliminate the release of ODS CFC / HC	41

4.4 - Technologies for the reduction of ground vibration	41

4.5 - Technologies for the reduction of energy consumption	42

Chapter 5 – BAT for treatment in shredders of fridges, freezers and air-conditioning units	42

5.1 – General BAT Conclusions	43

5.1.1 - Environmental Management Systems	43

5.1.2 - Monitoring	43

5.1.3 – Reception, handling and storage	45

5.1.4 - Reducing emissions to air	46

Chapter 6 – Emerging techniques	48

6.2 - Thermal Oxidation	48

6.3 – Dissolution of VOCs	48

Chapter 7 – Concluding remarks	49

Appendix 1 - Glossary	51

References	53

# Figures and tables

Figure 1: The effect on the ozone layer from the fluorocarbon refrigerants

Figure 2: Dimensions for twin-shaft shredder

Figure 3: Four shaft shredder

Figure 4: Details for four shaft shredder

Figure 5: Dimensions for twin-shaft shredder

Figure 6: Dimensions for super chopper

Figure 7: Views of the rotorshredder

Figure 8: Dimensions for rotorshredder

Figure 9: interior design of an air codition

Figure 10: Components of an air condition

Figure 11: A fully automated system for recycling refrigeration devices [12]

Figure 12: Tools and equipment for removing the CFC refrigerant

Figure 13: An  example  of an IR gas  monitor with the gas cell assembly exposed

Figure 14: Description of explosion protection measure

Figure 15: The PUR foam is separated out and transported to the Pelletizer/Pellet Press (PUR).

Figure 16: Magnetic separator belt

Figure 17:  Eddy current separator

Figure 18: Input materials, output materials and emissions of a refrigerator recycling plan

Figure 19: Input materials, output materials and emissions of a refrigerator recycling plan

Table 1: Specification for twin shaft shredder

Table 2 Specification for four shaft shredder

Table 3 Specification for super chopper

Table 4 Performance data

Table 5 Composition of an Average Refrigerator after 1996 [4]

Table 6: Explanation of the release points from figure 14

Table 7 The maximum mass loss of ODS per hour

Table 8 Monitoring parameters, applicability, point of measurement and monitoring frequency

Table 9 Maximum mass loss of ODS expressed as CFC per hour

Table 10 Level for dust

# Chapter 1 – General information

## 1.1 – The purpose of refrigerator recycling facilities

Fridge recycling is a key area of concern in the ongoing implementation of the European Directive (2012/19/EU) on waste electrical and electronic equipment (WEEE). It is not only the presence of hazardous chlorofluorocarbons contained in waste refrigerator and freezer appliances, but also the valuable  secondary raw materials that can be recovered from them that makes fridge recycling an extremely important area within the WEEE sector. Despite the fact that the first EU states to  implement fridge recycling did so back in 1988, and even though fridge recycling services are now available almost everywhere, the subject remains highly controversial.

Household refrigerators and freezers, depending on the manufacture date, contain different refrigerants in the cooling circuits and as blowing agents (in the insulation). Specifically, the fridges manufactured before 1995 typically contain chlorofluorocarbon (CFC) refrigerant and blowing agents. Most window air-conditioning units and dehumidifiers contain hydrochlorofluorocarbon (HCFC) refrigerant. CFCs and HCFCs are ozone-depleting substances (ODS) that, if released to the environment, they destroy the protective ozone layer above the earth. Ozone gas mainly exists in the stratosphere (8-10 km above the earth’s surface). The ozone layer absorbs most of the ultraviolet light from the sun, while letting light of different wavelengths through. A major contributor to ozone depletion is the range of fluorocarbon refrigerants and blowing agents, which include chlorofluorocarbons (CFCs) like R11 and R12 and hydrochlorofluorocarbons (HCFCs) such as R22. These synthetic gases are known to cause substantial damage to the ozone layer. The hydrofluorocarbon (HFC eg. R134a) range of refrigerants and blowing agents have no effect on the ozone layer, however they are synthetic  greenhouse gases and therefore contribute to global warming. All of these gases are used in refrigeration systems, insulation materials, air  conditioners, aerosols, solvents and in the production of some types of packaging[1].

Figure 1: The effect on the ozone layer from the fluorocarbon refrigerants

Moreover, CFC and HCFC refrigerants and blowing agent are also potent greenhouse gases, meaning that their release contributes to global climate change. Refrigerators and freezers manufactured since 1995 contain ozone-friendly hydrofluorocarbon (HFC) refrigerants; however, these refrigerants still need to be carefully handled since they are greenhouse gases [2]. Alternatives with no impact to the ozone layer and to the greenhouse effect are hydrocarbons (HCs[1]), such as R600a as refrigerant or cyclopentane as blowing agent.

The purpose of dedicated refrigerator and freezer recycling plant is to remove all refrigerants and all blowing agents from the waste appliances.  The operator of the treatment facility shall ensure that all removed VFCs and VHCs are captured and subsequently disposed of, or recovered, in accordance with requirements set by the European and national legislation and by international and national standard (f.e. EN 50574:2012).

Controversial point.3

[Proposal re destruction on site: Removed VFCs, VHCs and mixtures thereof shall either be transported to an authorised facility for recovery or disposal, or recovered or destroyed by onsite conversion.[19] or

Alternative proposal : Ozone-depleting gases must be treated in accordance with Regulation (EC) No 1005/1009 (Source: Directive 2012/19/ EU Annex VII No 2).]

It is also for these reasons that only the appliances containing ozone-depleting substances (ODS) or hydrofluorocarbon have to be processed using the techniques described in this document. Therefore, the equipment which are not containing ODS and HFC don’t fall within the scope of this Mini-BREF.

## 1.2 – The types of shredders in refrigerator recycling plants

### 1.2.1 Twin – Shaft shredder

The twin-shaft shredder is driven by a planetary gear reducer, and the blades are made of high tension alloy steel for a long working life, robustness, ease of adjustment and minimum maintenance, which have the advantage of high abrasion resistance and high strength. The two axes operated differentially, with functions of tearing, extrusion and occluding. The main focus of this twin-shaft shredder is shredding large diameter and tough material, with even and controllable output size. The twin-shaft shredding is characterized by low speed and high torque, capable of high throughput rates. The knives can be reversed when the shredder is blocked. The twin-shaft shredder is areliable equipment for size reduction in the refrigerator recycling industry.

[1] The HCs are part of the VHCs, defined in the EN 50574:2012 [19]

Figure 2: Dimensions for twin-shaft shredder

Table 1: Specification for twin shaft shredder

### 1.2.2 Four-shaft shredder

Four shaft shredders have advantages in standardization and modularization design, and high interchangeability for their components. Their blades are made of special alloy steel with high strength, strong anti-wear and reparability. The drive part takes advantage of four motors, heavy planetary gear boxes to make the main shafts rotate at different speeds. Touch screen programmable controls are available, with video monitor systems and automatic overload detection and protection functions. They have low rotation speed, big torque and low noise characters.

Figure 3: Example of a four shaft shredder

| Power and driver: The shafts are driven by two electric motors through a planet – gear gearbox | Advanced design of four shafts: High throughput in one pass with low rotation speed, low noise, large torque and high efficiency. |
| --- | --- |
| Principle of shafts and screen insert | Screens: Screens sizes (15 – 60mm) available |

Figure 4: Details for four shaft shredder

Figure 5: Dimensions for twin-shaft shredder

Table 2 Specification for four shaft shredder

| Model ITEM | FS100120 | FS130120 | FS130180 |
| --- | --- | --- | --- |
| A(mm) | 1220 | 1200 | 1800 |
| B(mm) | 2850 | 3500 | 4100 |
| C(mm) | 1680 | 4670 | 5270 |
| E(mm) | 1180 | 1600 | 2190 |
| F(mm) | 1670 | 1750 | 1750 |
| G(mm) | 800 | 1500 | 1500 |
| H(mm) | 2550 | 2650 | 3940 |
| I(mm) | 1020 | 1300 | 1300 |
| Main blade rotation diameter (mm) | Ø320 | Ø430 | Ø430 |
| Assistant rotation diameter (mm) | Ø320 | Ø430 | Ø430 |
| Main axle rotation speed (rpm) | 15-255 | 10-15 | 12-20 |
| Main blade quantity (pcs) | 30 | 24 | 36 |
| Assistant blade quantity (pcs) | 30 | 24 | 36 |
| Blade (mm) | 40 | 50 | 50 |
| Motor power (kw) | 22+22 | 15+15+30+30 | 15+15+30+30 |
| Equipment weight (kg) | 5500 | 12500 | 13500 |

### 1.2.3 Super chopper

Super choppers are heavy duty pre-choppers designed as a first step in some refrigerator recycling processes. The features for this shredder are:

- Low speed, high torque enabling processing of tough material

- Powered directly by hydraulic motor and power unit enabling automatic reverse for overload protection thus minimizing knife damage

- Starts up when fully loaded with material – reducing downtime after shutdown

- Adjustable knife clearance optimizing the cutting function

- The knives can be reground multiple times on a “flat-bed” grinder ensuring long life and low operation costs

- The machine is designed for quick and easy change of knives

- Unique rotor manufactured and designed for minimum wear width height

Figure 6: Dimensions for super chopper

Table 3 Specification for super chopper

| Technical Spec. | SC1412/160 | SC2118/250 | SC2118 II |
| --- | --- | --- | --- |
| Electrical motor | 160kW | 250kW | 500kW |
| Motor | CB400 | CB5560 | CB560 (x2) |
| Rotor | 1400mm/0-28 rpm | 2100mm/0-21 rpm | 2100mm/0-21 rpm (x2) |
| Knives | 12 flying/7static | 18 flying / 10 static | 18 flying/10static (x2) |
| Capacity/hr | Up to 12000 kg/hour | Up to 20000 kg/hour | Up to 15000–40000 kg/hour |
| Weight | Approx 15000kg | Approx 24000kg | Approx 35000kg |
| Length x width x height | 1700 x 3000 x 3500mm | 2000 x 3800 x 3800mm | 2800 x 4500 x 5000 mm |

### 1.2.4 Rotorshredder

Fridges are fed from the top into the Rotorshredder. These shredders t  can shred fridges up to 300kg depending on the feeding conditions. The chamber of the rotorshredder [3] contains rotating hammers fastened by articulated joints to a high-speed vertical shaft. They are arranged one above the other and are aligned horizontally and stabilized by centrifugal force.  As the refrigeration units enter the range of the hammers, they are subjected to intense impact, punching and shear forces, crushing them into small pieces. Once the crushed pieces have reached the desired size, they exit the chamber through a sturdy slotted gate. The material is crushed in a very short time and without interruption, resulting in a high degree of energy efficiency.

Hammer lifetimes last between 50,000 to 80,000 refrigeration units, which ensures high plant availability and productivity. The rotating hammers do not need to be adjusted or sharpened because they do not cut, functioning by force of impact instead. Wear-related costs for crushing come to about 0.08 € per refrigerator unit.   The shredding occurs by the hammers punching pieces of the refrigerator to the outside of the chamber as long as the pieces are too big to fit through the adjustable bar grate. Shredded material is therefore sized and leaves the working chamber through the bar grate.

Figure 7: Views of the rotorshredder

Table 4 Performance data

| Type | Working chamber (diameter) | Working chamber (Height) | Feed opening inside dimensions 1) (HxW) | Drive power (max.) |
| --- | --- | --- | --- | --- |
| RS 1515 | 1,500 mm | 1,500mm | 1,500mm | 110 – 200kW |
| RS 2018 | 2,000mm | 1,800mm | 600 x 1,200 mm | 110 – 400kW |
| RS 3218 | 3,200 mm | 1,800mm | 1,500 x 1,500 mm | 110-400kW |

1)  Not suitable for crushing solid parts of high strength. The maximum input size is equivalent to about two thirds of the feed opening.

Table 5 Dimensions and weights

| Type | A2) | B2) | C | D | E | Weight |
| --- | --- | --- | --- | --- | --- | --- |
| RS 1515 | 4,500 | 5,600 | 2.600 | 2,200 | 1,800 | 19,000kg |
| RS 2018 | 5,000mm | 6,000mm | 3,100mm | 3,850 mm | 2,200mm | 30,600kg |
| RS 3218 | 5,700mm | 6,700mm | 3,000mm | 5,130mm | 2,430mm | 42,000kg |

2) Different data may apply depending on the feed hopper that is used.

3) Weight for standard design including feed and discharge hoppers.

Figure 8: Dimensions for rotorshredder

## 1.3 – General information for refrigeration devices

Only the appliances containing ozone-depleting substances (ODS) or hydrofluorocarbon (HFC) are included in this BREF.

### 1.3.1. General information for fridges

Household refrigerators and freezers manufactured before 1995 typically contain chlorofluorocarbon (CFC) refrigerants in the circuit and in the insulation as blowing agents. Most window air-conditioning units and dehumidifiers contain hydrochlorofluorocarbon (HCFC) refrigerant. CFCs and HCFCs are ozone-depleting substances (ODS) that, if released to the environment, destroy the protective ozone layer above the earth. Moreover, CFC and HCFC refrigerants are also potent greenhouse gases, meaning that their release contributes to global climate change. Refrigerators and freezers manufactured since 1995 contain ozone-friendly hydrofluorocarbon (HFC) refrigerants; however, these refrigerants still need to be carefully handled since they are greenhouse gases.

Since the mid 90ies, producers also used hydrocarbons (HCs) such as R 600a (as refrigerants) and Cyclo-Pentane or Iso-Pentane as blowing agents.

Table 5 shows the composition of the components for fridges manufactured after 1996, measured in kg.

Table 5 Composition of an Average Refrigerator after 1996 [4]

| Component | Quantity per Refrigerator (kg) |
| --- | --- |
| Ferrous Metal | 30 to 40 kg |
| Non Ferrous Metal | 2 to 5 kg |
| Plastic | 5 to 8 kg |
| Glass | 1 to 2 kg |
| HFC – 134a Refrigerant | 0.06 to 0,15 kg |
| HC - R600a | 0.04 to 0.12 kg |
| Polyurethane foam insulation | 3 to 8.5 Kg |
| Pentane | 0.12 to 0.32 Kg |
| Oil (which may be contaminated with refrigerant) | 0.16 to 0.25 kg |
| Mercury | 9,07×10-4 kg |

### 1.3.2 General information for air condition

Most air conditioning devices typically contain freon as the cooling agent. Freon is a stable non-flammable moderately toxic gas or liquid which has typically been used as refrigerants . Refrigerants include the chlorofluorocarbons (CFCs) implicated in ozone depletion, but also include newer refrigerants which typically include fluorine instead of chlorine and do not deplete the ozone layer.

An air conditioner consists of four main sections,

- Refrigerant circuit components: compressor, evaporation condenser, capillary tube etc

- Ventilation: Distribution of air ((indoors) Heat dissipation (outdoors) (impeller, fan motor, etc...)

- Electrical parts: Climate control (remote control, electronic circuit boards, electrical system, etc..).

- Other: Unit casing, air filter, etc..

Figure 9: interior design of an air conditioning unit

Function of the main components[5]:

- Compressor: Compresses the refrigerant from low pressure (low temperature) to high pressure (high temperature). This conversion raises the boiling point to higher temperature levels, facilitating elimination of the heat brought by the outdoor air.

- Condenser: This component receives gas at high pressure and high temperature from the compressor. In air-cooled condensers, the metallic surfaces cool the gas which changes status and turns to liquid. In the case of water-cooled condensers, it is the circulation of the water that produces the same cooling effect.

- Evaporator: When the refrigerant evaporates in the evaporator, it absorbs heat from the surrounding air and produces cooled air.

- Capillary tube: A narrowing of the tube connected along the line between the condenser and the evaporator with diameters ranging from 1 to 2 mm. and lengths ranging between 1 and 2 m, allows the adjustment of the amount of gas fed to the evaporator.

- Electrical parts: Electric and electronic components needed by the various air conditioner functions.

- Indoor fan: It exhausts air from the indoor environment and conveys it through the evaporator; the air is now cool and distributed back into the environment.

- Outdoor fan: This causes the air to circulate through the condenser in order to cool the refrigerant.

| Compressor | Condenser | Evaporator |
| --- | --- | --- |
| Electrical parts | Indoor fan | Outdoor fan |

Figure 10: Components of an air condition

### 1.3.3 Refrigerants and blowing agents

It is specified in 2012/19/EU WEEE Directive (Annex VII) that chlorofluorocarbons (CFC), hydrochlorofluorocarbons (HCFC) or hydrofluorocarbons (HFC), hydrocarbons (HC) have to be removed from any separately collected WEEE.

Also in 2012/19/UE WEEE Directive, it is specified that equipment containing gases that are ozone depleting or have a global warming potential (GWP) above 15, such as those contained in foams and refrigeration circuits: the gases must be properly extracted and properly treated. Ozone-depleting gases must be treated in accordance with Regulation (EC) No 1005/2009.

Fluorocarbon refrigerant gases and blowing agents must be safely removed from any equipment that has reached the end of its service life.

About CFCs: CFC refers to the chemical composition of the refrigerant. ChloroFluoroCarbon indicates that the refrigerant is comprised of Chlorine, Fluorine, and Carbon. A chlorofluorocarbon (CFC) is an organic compound that contains only carbon, chlorine, and fluorine, produced as a volatile derivative of methane, ethane, and propane. They are also commonly known by the DuPont brand name Freon. The most common representative is dichlorodifluoromethane (R-12 or Freon-12). A blend that contains a CFC and any other product is still considered a CFC refrigerant. These substances harm public health and the environment by destroying ozone in the upper atmosphere. The most common CFCs refrigerants are R11 and R 12.

About HFCs: HFC refers to the chemical composition of the refrigerant. HydroFluoroCarbon indicates that the refrigerant is comprised of Hydrogen, Fluorine, and Carbon. A blend that contains different HFCs is considered an HFC refrigerant. An example of HFCs refrigerant is R-134a. It is a haloalkane refrigerant with thermodynamic properties similar to R-12 (dichlorodifluoromethane) but with less ozone depletion potential. It has the formula CH2FCF3.

About HCFCs: HCFC refers to the chemical composition of the refrigerant. HydroChloroFluoroCarbon indicates that the refrigerant is comprised of Hydrogen, Chlorine, Fluorine, and Carbon. A blend that contains both an HCFC and HFC is considered an HCFC refrigerant. An HCFC substance harms public health and the environment by destroying ozone in the upper atmosphere The most common HFCs refrigerant is R22 (Chlorodifluoromethane). As it contains Chlorodifluoromethane , it harms public health and the  environment by destroying ozone in the upper atmosphere[6].

About HCs: The hydrocarbon chemicals Ethane, Propane, Isobutane, Butane, Ethylene, Pentane (N-Pentane, Cyclo-Pentane and Iso-Pentane) are also included in the category of refrigerants and blowing agents.

One of the main characteristics, in comparison with CFC, HCFC and HFC is that HC has no ozone depleting potential (ODP) and only a low global warming potential (GWP) [24]

Environmental impacts of blowing agents in PUR-foam.

More references [25]

## 1.4 The process

The quality assurance specifications for the dismantling of CFC-containing refrigerators and freezer appliances focus on the two main stages of refrigerator recycling. The first stage involves the collection and storage of the waste appliances, the second stage is concerned with their processing. The processing stage is itself further divided into Stage 1 (extraction of CFC and HC refrigerants and refrigerator oil from the cooling circuit), Stage 2 (extraction of CFC and HC blowing agents from the insulating material) and finally, the handling of the output material streams from Stage 1 and Stage 2[8].

Before the refrigerators are broken up, the loose inner parts are taken out, this includes glass, racks, wood, and if present, the capacitors and mercury switches.  This dismantling operation is follοwed by removing of the oil and CFC (e.g., R12) / HCs (e.g., R600a) / Iso-butane that is done with the refrigerant suction system. Then the compressor is removed from the refrigeration device and the body of the fridge is then moved to the fridge shredder. The refrigerators are automatically moved by hydraulic lifts or belt conveyor to the shredder. These feed materials then fall by gravity into the shredder box. Within the shredder box is a heavy, fast-spinning rotor equipped with hammers which cut the material on anvils within the shredder box and then drive the shredded materials through sizing grates to enable further process of the scrap to the desired state of separation of one material from another. During the shredding of the refrigerators  the gasses trapped in the shredder chamber are removed and depending on the technology, there are different methods to eliminate the risk of fire and explosion and manage the CFC/pentane/air-mixture from shredder unit.

The mixed shredded materials are then transported to a range of separation systems which separate the shredded materials into streams of ferrous, mixed non-ferrous scrap, foam and plastics. In general the output size of the materials coming outfrom a shredder is approximately 20 to 30 mm.

Each step of the process has to be monitored meticulously. According to EN 50574:2012 any input and output material have to be monitored on a daily base and reported on a monthly base [19].

## 1.5 – Refrigerator recycling plants in Europe

Every year in Europe, millions of refrigerators reach their end-of-life. The discussions on climate change have seen increasing focus on effects of chlorofluorocarbons on the climate: CFCs are powerful greenhouse gases and make a substantial contribution to global warming.

According to the IPCC (Intergovernmental Panel on Climate Change), the following carbon dioxide equivalents should be used for the CFCs found in end-of-life refrigeration equipment [9]:

• 1 kg CFC R12 is equivalent to 10,900 kg of CO2

• 1 kg CFC R11 is equivalent to 4,750 kg of CO2

A European refrigerator, containing on average 125 grams of CFC R12 in its cooling circuit and about 312 grams of CFC R11 in the insulation materials, has a GWP equivalent to around 2.8 metric tons of CO2. Experts have calculated that this is equivalent to the amount of CO2 emitted by a medium-sized European car travelling about 17,000km. The estimated 200 million CFC-containing refrigerators and freezers still in use in Europe today therefore contain the equivalent of about 560 million metric tons of CO2. These staggeringly large numbers demonstrate just how important it is to recover every gram of CFC when recycling waste refrigeration equipment. Experience in Europe over the last few years has clearly shown that compliance with quality assurance standards is essential if recycling companies are to be able to guarantee the maximum possible recovery of CFCs from waste refrigeration equipment.

With a population of around 450 million in the EU, around 18 million waste fridges and freezers are generated every year and treated. This is equivalent to over 630,000 tonnes of recyclable refrigeration equipment per day. To handle this load, some 60 fridge recycling plants operate in Europe.

Today, there are currently more than fifty full-process fridge recycling plants operating in Europe. These plants are manufactured by a very small group of plant engineering and construction companies [9]. Today, in Europe, there are less than 10 construction companies [10], which are specialized in designing, manufacturing, sales associated to the handling, processing and recycling of refrigeration devices enabling facilities to comply with the recast Directive 2012/19/EU on waste electrical and electronic equipment (WEEE). Never the less each company promotes its own technology, which differ in certain stages of the recycling process.

## 1.6 - Ecological and economic aspects of the refrigerator recycling process

The full recovery of CFCs from waste fridges and freezers is a very important environmental and climate protection issue. The recycling of refrigeration devices, in a full equipped plant, could lead to a reduction of release of chlorofluorocarbon in the atmosphere, over than 15,000 tonnes for the next 10 years [11].

Moreover, from the recycling process, approximately 63kg of metal and 11.33kg of plastic is recycled per appliance. As a result a recycling unit could recover 93% of metal and 100% of plastic [4].

Special advantages of shredder scrap

In comparison to coarser scrap, the density of shredded scrap is up to 50% higher. The higher density means that the scrap basket can be filled with a greater quantity. An initial charge of shredded ferrous scrap provides a protection to the refractory of the Electric Arc Furnace (EAF) prior to loading heavy scrap. Shredded ferrous scrap allows the electrodes to penetrate the charge better and so improves start-up. Furthermore, during the melting cycle shredded scrap can be charged into the Electric Arc Furnace using a conveyor without lifting the furnace roof, so reducing emissions and energy consumption. The steel yield per tap is thus increased and overall less scrap has to be charged. In addition, the tap-to-tap cycle is shortened. Another ecological benefit is that the roof of the Electric Arc Furnace does not have to be opened as often in relation to the number of melting processes, thus allowing greater energy conservation by 3 to 7%. Moreover, production can be increased by up to 18% a year due to the higher input of scrap that is possible. Due to the material and/or energetic utilisation of the metals, residual fractions and residues, landfill space is no longer used and the emissions typical of such no longer occur.

In its report from the year 2008 the BIR (Bureau of International Recycling) looked into the environmental benefits that are achieved by recycling and determined the values for saving energy and CO2, given below, when recycled ferrous metals are used:

a) Energy conservation in the production of 100,000 tonnes of steel, depending on the method:

- primary campaign production in the blast furnace/converter: 1,400 TJ

- primary production DRI-EAF 1,920 TJ

- secondary production EAF 1,170 TJ

b) CO2 balance in the production of 100,000 tonnes of steel, depending on the method:

- primary campaign production in the blast furnace/converter: 167,000 t CO2

- primary production DRI-EAF 70,000 t CO2

- secondary production EAF 70,000 t CO2

1,784 kWh per tonne is needed to produce one tonne of liquid steel from 100% scrap. Theoretically, 6,481 kWh per tonne is needed to produce one tonne of crude steel from primary raw materials. The amount of energy saved is around 4,697 kWh per tonne, i.e. approximately 72.5 %.

## 1.7 - Economic and institutional aspects of the refrigerator recycling process

Based on European or national regulations, permits are generally delivered by local authorities considering the characteristics of the plant location and the nature of its industrial activities. Various requirements are generally specified concerning:

- Type and size of the activities on the shredder yards

- Energy supply

- Definition of controls from the administration

- Action taken to prevent water pollution, definition of used water collection constraints

- Action taken to prevent fire risks

- Definition of chemical requirements on water sent back to the environment

- Action taken to prevent air pollution

- Definition of chemical and physical requirements on air emissions

- Waste management

- Noise and vibration limits

## 1.8 – General environmental issues related to shredders

In this section we give an overview of the environmental aspects that may play a role at refrigerator recycling plants.

### 1.8.1 – Waste

The improper storage of waste can lead to contamination of soil and groundwater, surface water and air, if not prevented.

Those following substances or materials result from operations at the refrigerator recycling plant :

- Iron and steel scrap (may be classified as a waste, or as a product if compliant with Regulation (EU) No 333/2011).

- Non – ferrous scrap, aluminum and copper fractions (may be classified as a waste, or as a product if compliant appropriately with Regulation (EU) No 333/2011 or Regulation (EU) No 715/2013).

- Plastic fraction, this is principally polystyrene and polyurethane that can be reused as secondary raw materials

- PUR foam into pellets or dust.. Each refrigerator contains approximately 4.5 kg. of polyurethane foam [4]. The polyurethane granulate extracted from the refrigerators is a good oil binding agent.

- Shredder light fraction (SLF) comprised of dust from the cyclone, fabric filter, and a mix of plastics, fibres and mineral fraction sorted during the cleaning of shredded scraps by the windsifter (drum or zigzag)

- Oil and CFC/ HC refrigerants and blowing agents.

### 1.8.2 - Use of energy

Electrical energy is used for powering the tools and equipment, particularly the shredder.

### 1.8.3 - Contamination of soil and ground water

Contamination of soil and groundwater could occur in the case of:

- A poorly maintained impermeable site surface

- A poorly maintained or inadequately sized drainage system and/or oil-water separator

Pollutants in the soil and/or groundwater from any such occurrences could include:

- Non-volatile organochlorine compounds (including polychlorinated biphenyls (PCBs),

- Compressor oil

- Hydrocarbons [2][4]

### 1.8.4 – Pollution of air

Air pollution that could occur under normal operating conditions:

- The emission of dust could occur when handling and storing scrap

- Dust or water vapor from the shredding process or the air exhaust

- Release of CFCs/HCs.

# Chapter 2 – Applied processes and techniques

First, an outline of a refrigerator recycling plan is given to describe the applied processes and techniques. There are several plant areas to differentiate between, but the ones that are relevant for this document are:

- Delivery and acceptance

- Pre-treatment

- Stage 1: Extraction of refrigerants and refrigerator oil

- Stage 2: Treatment of the drained refrigerators

Figure 11: A fully automated system for recycling refrigeration devices [12]

## 2.1 – Common techniques applied to the sector

### 2.1.1 – Delivery and acceptance

All sites for storage shall at least be in line with the technical requirements of Directive 2012/19/EU [and the EN 50574:2012 – see Controversial point 2].

- Impermeable surfaces for appropriate areas with the provision of spillage collection facilities and, where appropriate, decanters and cleanser-degreasers,

- Weatherproof covering for appropriate areas.

Operators will normally designate a material reception area, with trained staff controlling the inspection, reception and validation of materials at the installation and establish quarantine areas for materials that are prohibited, awaiting full inspection, testing or removal. Fridges should be checked on arrival for damage or leaks. Those in a poor state of repair or which pose a significant risk of releasing CFC should be de-gassed immediately.

The delivery area of a refrigerator recycling plant consists of a closed and sufficiently sized acceptance area, which as a rule is a paved surface with a weighing platform (weighbridge) and corresponding weighbridge office. The weight of all material input and output streams are recorded at the weighing station.

The delivery area offers the possibility to carry out an intake control, for both commercial and legal reasons with regard to the waste, and to make a pre-selection according to visually recognizable criteria.

The input material delivered to a refrigerator recycling plan is called feed. As a rule, a recycling unit for refrigeration devices accepts as feed household appliances, which contain CFC, HCFC, HC and HFC refrigerants and blowing agent. The shredder feed comprises:

- fridges

- air-conditioning units

- freezer appliances

- dehumidifiers

As it is mentioned in the Chapter 1.1 the classification of refrigerators is made ​​by the production date. So the proportion of the refrigerators in the total volume is difficult to calculate since the composition of the shredder feed varies from one plant to another and depends on the season and region.

### 2.1.2 – Pre-treatment

Before the refrigerators are broken up, external electrical cables, the loose inner parts including glass, plastic racks etc. are taken out, and – if present, the capacitors and the mercury switches are removed. and printed circuit boards greater than 10 cm².

### 2.1.3 - Stage 1:  Extraction of the refrigerants and refrigerator oil

This operation is follοwed by the removal of the oil and refrigerant (CFC/Iso-butane-mixture) that is done with the refrigerant suction system. A refrigerant recovery unit is used to pump fluorocarbon and hydrocarbons refrigerant out of refrigeration and air conditioning systems and into a recovery cylinder. Stationary units such as refrigerators and small air conditioners may not have a service hose access port (service valve). In such case the refrigerant recovery is achieved by using approved piercing pliers or a drill head. Once the line is pierced with the tool a small valve prevents the flow of refrigerant. Best practice guidelines recommend that service hoses are fitted with ‘ball’ valves at one end. Ball valves prevent the accidental release of fluorocarbons or hydrocarbons refrigerant into the atmosphere and assist in the safe removal of the service hoses. When the ball valve handle is parallel to the hose refrigerant will flow. When the ball valve handle is at right angles to the hose refrigerant cannot flow. Once your blue service hose is connected to the tool, fluorocarbon or hydrocarbons  refrigerant will flow to the recovery unit. A commonly available set of piercing pliers is shown below[1].

| A refrigerant recovery cylinder | Service hoses with ball valves closed |
| --- | --- |
| Piercing pliers |  |

Figure 12: Tools and equipment for removing the CFC / HC refrigerant

Then the compressor is removed from the refrigerator  and passed on for further processing. After removing the compressor the refrigerator is automatically moved to the fridge recycling system.

To achieve the best possible outcome refrigerant and oil should be extracted in the same step using vacuum suction [12]. The reason for this is that the initial pressure of the system is the most effective way of removing the oil. Once the pressure is lost, maximum suction will only produce 1 bar of negative pressure.

Requirements for de-gassing processes [12]

There are two options for de-gassing

- Removal of refrigerant and oil in the same step for example piercing the base of the compressor with a proprietary degassing unit and extracting refrigerant and oil

- Removal of refrigerant and oil in separate stages for example “nipping” the refrigerant circuit and extracting the refrigerant followed by attaching a tube to the base of the compressor via the stem of the cooling circuit and pumping out the oil.]

Option A

Proprietary degassing units are capable of removing > 99% (> 90 %) of oil and refrigerant mix from the cooling circuit and compressor at typical temperatures of 5-10 degrees Celsius. Prior to de-gassing the temperature of the oil must be at least 5 oC.

Option B

The extraction steps should be carried out in an enclosed area where emissions can be captured and the controlled substances released recovered (by for example adsorption or condensation) for disposal. The extraction steps should be carried out in immediate sequence without delay in between. Emissions of ODS should be limited to a maximum of 5g per unit from the de-gassing process (as assessed from the captured refrigerant in the controlled environment).  Air-flows need to be managed in such a way as to avoid the development of dangerous atmospheric concentrations of refrigerant. This could happen, for example, through the degassing of hydrocarbon refrigerators.]

In order to reduce the viscosity of the oil prior to de-gassing the temperature of the oil must be brought up to a temperature of 20 oC prior to de-gassing. When oil/refrigerant mixes are extracted, the fractions should be separated as quickly as possible to allow for maximum extraction of refrigerant from the oil.

Achievable performance [12]

Gaseous refrigerants are harder to contain than liquid blowing agents. Option A has the best performance with an achievable ODS loss of 1.17g per unit from the refrigerant and 0.14g of ODS in the oil. Under any circumstances the losses should not exceed the range of 1.5-3.5g per unit.

### 2.1.4 - Stage 2: treatment of the drained refrigerators

2.1.4.1 Fully automated systems

A typical fully-automated plant will handle between 60 and 100 (35 and 45) refrigeration devices per hour. The refrigerators are automatically moved by hydraulic lifting and/or belt conveyor to the shredding unit. The cutting operation takes place in a shredding chamber, where the concentration of gases is controlled to eliminate the risk of explosion, e.g. limiting the concentration of pentane. Gases such as  CFCs and HCs used as foaming agentscould be released as from the insulting foam of the scrapped domestic appliances [13].

Figure 13: An example of an IR gas monitor with the gas cell assembly exposed

As far as concerned the explosion protection measures the concentration of pentane may be monitored automatically  (by IR sensors at different measuring points - redundancy) set to the LEL (Lower explosion level) in the filling hopper [14]. The detection of pentane using IR has been available for a few years. However, because of many design options, the features and implementation of the technology vary greatly from one manufacturer to another.

The following limit values are adjusted [14]:

- 0% - <10%: no action

- >10% - <30%: add nitrogen directly out of the tank installation via regulating valve; flow volume will be monitored

- >30% - <40%: stop the feeding, close the hopper flap. Extraction system continues; maybe increase extraction

- >40%: Emergency stop, stop shredding, extract system continues, nitrogen feeding continues.

If the value subsequently falls:

- <30% plant continues to run

- <10% close the nitrogen valve.

There are also two delivery points for nitrogen in the filling hopper which are positioned at different spots. The filling hopper has an entrance for maintenance work and a smaller revision door which is equipped with safety limit switches [14].

In most cases the refrigerator recycling units use systems which inject nitrogen constantly into shredder, the crusher, and the dust collector in order to prevent explosions.  At the same time, water sprays are used to prevent high temperatures in the shredder and crusher [15]. The figure below shows the safety measures for explosions.

Figure 14: Description of explosion protection measure

Before the shredded pieces are extracted from the shredder chamber, the harmful gasses trapped in the shredding chamber are automatically pumped out and cooled down with liquid nitrogen to minus 90 degrees Celsius. At that freezing temperature, the gasses turn into liquids. The liquefied gasses are stored in tanks. The shredded materials are sent to a Zig-Zag Wind Separator [8]. At this step the shredded pieces of PUR-foam are separated by the wind sifting and are moved into the buffer silo of the pelleting press where they are compressed to firm pellets. The powdery polyurethane insulating foam (polyethane granulate) is heated to around 110 oC in a close heating spiral to extract the entire remaining expanding agent [14]. The exhaust air from this process step is also pumped into the process gas cleaning System.

Alternatively the PUR foam pieces are ground in a mill system. To degas the blowing agent out of this PUR-powder, the powder is heated in a vessel for about 4 hours.[ 22 ]

Other recovery systems have been developed to ensure the recovery of CFC/HC-containing insulation and blowing agents that are released from the insulation foam during shredding.

Figure 15: The PUR foam is separated out and transported to the Pelletizer/Pellet Press (PUR).

The rest of the shredded materials are led to an overband magnet, in order to separate the ferrous particles from the other materials (aluminum, copper and plastic fractions). The remaining fraction is passed over the Eddy Current Separator, where it is separated into plastic and aluminum/Cu fractions [8]. An eddy current separator uses a powerful magnetic field to separate non-ferrous metals from waste after all ferrous metals have been removed previously. After these separation processes the separate fractions are stored prior to sale[14].

Figure 16: Magnetic separator belt

Figure 17:  Eddy current separator

#### 2.1.4.2 Semi-automated and manual processing

Manual processes are either classified as [12]:

(1) The manual dismantling of the cabinet taking place as a discreet operation, with the foam components being shipped elsewhere for further processing in a semi-automated or fully-automated plant, or

(2) The manual dismantling of the cabinet taking place as a pre-cursor to the direct incineration of the foam and other components.

The principle of reducing ODS loss to a minimum is equally important to this type of processing. It presumes that refrigerator and freezer cabinets can be manually dismantled particularly in order to ensure that losses of foam on the metal and plastic components of the cabinet are kept to a minimum. Polyurethane is a natural adhesive and this characteristic is part of the requirement of the cabinet design. Given that the blowing agent is, to a significant extent, dissolved in the polyurethane matrix, there is no strictly manual process by which the blowing agent can be recovered and destroyed. Unless the operator can demonstrate that semi-automated or manual process can meet the performance requirements stipulated for fully automated units, semi-automated and manual processing should not be implemented.

## 2.2 – Emission reduction technologies at refrigerator recycling plant

The application of the existing legal standards for reducing emissions is handled differently by the authorities depending on the country, the region, and the location of the shredder site. That means that the technologies and processes used vary from site to site. Below are descriptions of individual measures that have been implemented at various locations in European Countries. The principle of proportionality must be observed when selecting the emission reduction technologies.

### 2.2.1 – Technical measures for degasification inside the shredding area

As far as concerned the degasification, which take place inside the shredder unit, there are four processes that are known:

- The nitrogen process [15]: The Shredding area is under air suction, CFC/Pentane is exhausted and N2 is blown in to reduce the O2 level to <4%. There is no risk of fire when the O2 is below 4%. For a shredding chamber, size of 2000x1200x1000 mm, the total amount of air exhausted would be 400m3/hour. The CFC/Pentane is then exhausted inside the unit, which applies a chemical treatment1. In this stage, the CFC/Pentane is separated from the N2 and the water. The CFC/Pentane is fed to a collecting tank for fluid CFC/Pentane. The tank is placed on a scale, which makes it possible to see how much gas is recovered and when the tank has to be changed. The N2 is recovered and send back into the Shredding area and the water is recovered and send into the drain.

- The IC process2 (Impact Catalyst)[16]: The shredding area is supplied with air continuously, so the concentration of pentane always remains far below the lower explosive limit (LEL). For this reason, inerting of the shredding area is no longer necessary. Gas purification takes place in two catalytic converters connected in series. The hydrocarbon compounds such as isobutane and pentane are oxidized in the first converter to form water and carbon dioxide. In the second reactor, the CFCs are converted into the components hydrogen, chloride (HCl), hydrogen fluoride (HF), and carbon dioxide (CO2). The HCl and HF gases are converted via washer stages into a concentrated saline solution, which must be disposed of externally.

- Cryogenic method [19]: The recycling machine automatically pumps out the harmful gasses trapped in the shredding chamber and cools them down with liquid nitrogen to minus 90 degrees Celsius. At that freezing temperature the gasses turn into liquids. The liquefied gasses are then stored in tanks.

- Recovery of CFCs from foam [18]:  The shredding takes place in a closed system to ensure the recovery of environmentally harmful CFC/HC-containing insulation and blowing agents that are released from the insulation foam during shredding.

### 2.2.2 – Technical measures for the reduction of dust in exhaust air

#### 2.2.2.1 - Bag filters

The dusty exhaust air is passed through the bag filters [15] or fabric filters, where a porous filter layer allows the gas through and leaves the dust particles on its surface. The filter fabrics are arranged as bags in a housing that is divided into several chambers. The filters work without moisture so that periodic shaking or blasts of compressed air can dedust the filter material. Their use in the main exhaust air flow presupposes that the danger of deflagrations resulting from the rotor chamber is virtually zero. Sustained operation of such a separation stage is otherwise impossible As with every deflagration the bags would be destroyed by the pressure surge or – if fire develops – by the temperatures. At the same time, water sprays are used to prevent high temperatures in the shredder and crusher, in order to prevent fire or explosions.

Status: Bag filters can be used at industrial shredder plants with separated air systems and separate exhaust air lines (e.g. air separator dedusting).

### 2.2.3 – Technical measures for the reduction of gaseous organic compound in exhaust air

#### 2.2.3.1 - Activated carbon filters

Activated carbon filters [18] belong to adsorption filtration technology, by means of which gases and vapors are adsorbed independently on the surface of solid materials due to physical or chemical reactions. The quantity of adsorbed material depends on the surface and surface structure. Activated carbon has a very large internal surface and is able, by the use of different coatings, to bind all harmful gases. If the basic conditions are right, activated carbon tends to auto-ignite. Furthermore, dust fractions in the exhaust gas flow can clog the filter bed prematurely, i.e. before its intake capacity has been exhausted, making its replacement necessary

#### 2.2.3.2 - Thermal Oxidation

The aim of thermal oxidation is to destroy pollutants in the air that can oxidize. Taking in to consideration IC process [16] combining mechanical crushing of the fridges with thermal-catalytic oxidation of the refrigerants and blowing agents. The hydrocarbon compounds such as isobutene and pentane are oxidized in the first converter to form water and carbon dioxide. In the second reactor, the CFCs are converted into the components hydrogen chloride (HCl), hydrogen fluoride (HF), and carbon dioxide (CO2).

Status: Generally Thermal Oxidation is energy intensive and questionable in economic and resource efficiency terms.

### 2.2.4 – Technical measures for the reduction of noise

Specialised fridge recycling plants are commonly housed in buildings. The main mitigation/attenuation measures for noise are distance and screening. When considering the layout of an installation it is unlikely that the relocation of certain operations within a site would achieve a significant reduction in noise at the site boundary. For example, an installation located 100m from a sensitive receptor would have to be located a minimum of 80m further away to achieve any significant (perceptible) noise reduction. The limited ability for relocation on many installations means screening and operational/management changes may be necessary to control noise. Local screening and localized noise barriers placed around particular activities or processes would achieve significant noise reductions. With use of the appropriate noise absorbing materials this could be 5-10dB on the screened side.

### 2.2.5 – Technical measures for water treatment

Most stored refrigerators collect water and up to 1 kg/unit can be extracted [12]. Existing processors sometimes handle this water by collecting into the same “pot” as that used for the recovered ODS. However, this can be an expensive route via special waste incinerators. Accordingly, others separate the water and have it analyzed in order to establish how it should be treated.

# Chapter 3 – Current consumption and emission levels

The consumption and emission of a fridge shredder plant depends to a great extend on the shredder technology used and the respective downstream emission reduction technologies used. The essential input materials and emissions of a fridge shredder plant are shown below.

Figure 18: Input materials, output materials and emissions of a refrigerator recycling plan

## 3.1 Energy input and auxiliary materials

The energy input in the form of electrical power depends directly on the rotor power and the type of processes and technologies used. It is thus very specific to each individual case and only comparable to a limited extent. Gross monitoring and reporting of total consumption may be undertaken for cost purposes, and, for example, the rate at which the mill draws current on site may be monitored live, but it may not be recorded.

The configuration of a refrigeration recycling plant is such that the power used by the downstream separation and segregation components, and the conveyors that feed them, is typically fixed. The separation methodology is calibrated to deal with an average/ideal infeed rate and material type. There is some flexibility in the separation processes, i.e. flexibility to increase the plant suction, adjust conveyor speed though the magnets and eddy current magnets to deal with changes in the feed quality, quantity and environmental conditions at the time. The key to controlling and minimising the power usage is down to the fragmentising operation. The process requires that the mill chamber/box is full and the feed is consistent. The mill under these circumstances would pull a consistent current. If the feed is ‘lumpy’ then the rotor will slow down during heavy loading and speed up during light loading. In the situation of periodic heavy loading the rotor slows, losing momentum due to the greater resistance of the material. This requires more power from the motor to increase the rotor speed and replace the lost momentum. The efficiency of the mill is also reduced when it is left running empty or the load rate falls below the optimum.

Manufacturer information suggests the power use estimates are based upon the size (infeed and mill box size) and power rating of the mill and its production capacity.

Power consumption figures (per tonne of material processed, kWh t-1): the average power consumption was 27.2 kWh t-1 with a range of 17 kWh t-1 to 47 kWh t-1.

Water consumption rate (per tonne of material processed, m3t-1): the average rate of water consumption was 0.032 m3 t-1 with a range of 0.001 m3 t-1 to 0.12 m3 t-1.

Note: those above data are specific for the Mewa or BHS process.

The other auxiliary materials used are mainly fuels and lubricants for grabs, industrial trucks and in-plant transport. The forms of use and specific consumption in this field are so varied that it is not possible to generalize.

## 3.2 – Water Emissions

According to the figure below water may be added to the mill in order to prevent high temperatures in the shredder and crusher.  The objectives are to reduce the quantity of any water to be discharged from the installation, and to reduce the degree to which these waters are exposed to potential contaminants.

Figure 19: Input materials, output materials and emissions of a refrigerator recycling plan

Where water is not reused, preference for discharge waters should be to foul sewer. Only where locations or other engineering constraints prevent connection to foul sewer should the discharge be connected to controlled water. Connection of discharge to controlled water may require additional treatment to meet the consent limits.

Taking into consideration the IC process [16], a question is raised about the externally disposed of HF and HCl and the danger of contamination of water.

## 3.3 – Air emissions

Emissions to air from a shredder plant are likely to be dust, (ODS)  CFCs/HCs, water vapor or, occasionally, smoke in the event of a deflagration.

### 3.3.1 - Dust

Tests demonstrated that the general emission limit values for dust (50 mg/Nm ³) can be well respected for all installations. The total of listed metal emission rates largely follow the particulate levels

### 3.3.2 ODS

An emission limit value of 150 mg/m3 g CFC R11 equates to a mass loss of 75g/hr. The mass loss of ODS from the system should be proportional to the feed rate of appliances into the system. Therefore the mass loss should not exceed these values [12]:

Table 7 The maximum mass loss of ODS per hour

| No. of refrigerant units processed per hour | Maximum mass loss of ODS expressed as CFC R11 |
| --- | --- |
| <100 | 5 g/hr |
| >100 but <200 | 10 g/hr |
| >200 but <300 | 15g per hour |
| >300 but <400 | 20g/hr |

### 3.3.3 – PCB

PCBs have been prohibited from use since 1987 in Europe. Nevertheless the infeed of a recycling plant can occasionally contain used equipment sold on the market before 1987, in which traces of PCB can be found. Taking this fact into account means that any capacitor has to be removed regardless of its potentially contained substances. It would make no sense to sort out “clean” capacitors from contaminated capacitors. The amount of Dioxin-Like PCB (DL-PCB) entering shredders is decreasing and will continue to decrease in the future.

### 3.3.4 - Mercury

Mercury can arise as mercury switches of fridges and freezers. Directive 2012/19/EU on waste electrical and electronic equipment (WEEE), Annex VII, Section 1 header and  indent 2: requires “As a minimum the following substances, mixtures and components have to be removed from any separately collected WEEE: — mercury containing components, such as switches or backlighting lamps, 3.4 – Noise

The noise emissions occurring in a refrigerator recycling plants are primarily caused by the machinery and materials handling technology as well as the delivery of the shredder feed and the transport of the products. The dB is a logarithmic scale used to describe sound pressure and sound intensity. Therefore a change of 10dB represents an equivalent change by a factor of 10 in the sound pressure, subjectively, this might be perceived as a doubling of the loudness of sound. Monitoring has been carried out in some Member States of both shredder operational/processing levels and idle non-processing levels, and levels associated with the installation when the mill was shut down. Such information indicates that the mill is the main source of noise on the site even though the other operations would generate a significant contribution to the plant noise. It should be noted, however, that it is not just the level of noise which is significant, but the characteristics of how and when the noise is produced; so for the outfall conveyor, the noise tends to be higher pitched. Furthermore, continuous noise tends to be less intrusive and less noticeable than irregular events. This may mean that tipping is a more noticeable intrusion than the continuous sound of the mill. This may be more significant when the average level of tipping and feed conveyor area was found to be only 3dB below that of the mill.

Notwithstanding that refrigerator shredders are commonly housed in a building, sound barriers can be erected in certain cases to minimize the noise emissions reaching neighbouring areas where studies indicate a positive noise prognosis. The main mitigation/attenuation measures for noise are distance and screening. When considering the layout of an installation it is unlikely that the relocation of certain operations within a site would achieve a significant reduction in noise at the site boundary. For example: an installation located 100m from a sensitive receptor would have to be located a minimum of 80m further away to achieve any significant (perceptible) noise reduction.

Because the refrigerator recycling process is a close design process, the sound levels are lower than the typically open shredder plant. Still the limited ability for relocation on many installations means screening and operational/management changes may be necessary to control noise. Local screening and localised noise barriers placed around particular activities or processes would achieve significant noise reductions. With use of the appropriate noise absorbing materials this could be 5-10dB on the screened side.

Vibrations can be prevented or minimized by proper balancing of the hammers in the mill chamber.

Noise limits may be prescribed in the permit conditions for the installation, typically measured at the site boundary, in the range of 60 and 80dB.

# Chapter 4 – Techniques to consider in the determination of BAT

This chapter should be adjusted according to the proposed amendments above.

The fact that the processing of old fridges always has to take place  in accordance with the best available technology means that plant  operators have to co-operate with an innovative and experienced  manufacturer. The CENELEC Standard (EN 50574:2012) places higher demands on the processing of old fridges as of 2013 and poses a  big challenge - for both the manufacturer of the system as well as  its operator. This CENELEC standard defines the requirements in the terms of the handling, transport, storage, sorting and treatment of WEEE domestic appliances that contain volatile chlorofluorocarbons, volatile hydrocarbons or both at the end of their lifecycle.

The CENELEC Standard (EN 50574:2012) and its Technical Specifications (TS EN 50574:2014) describes the requirements for the removal of volatile fluorocarbons (VFCs) and volatile hydrocarbons (VHCs). These substances can be found as refrigerants in the cooling system and as blowing agents in the insulting foam of the scrapped domestic appliances.

## 4.1 Delivery and acceptance

### 4.1.1 Transportation and storage

It is permitted to transport old refrigeration devices with other WEEE [19], as long as they are not damaged during the transportation. Temperature exchange equipment e.g. Refrigerators, Freezers, Equipment which automatically delivers cold products, Air conditioning equipment, Dehumidifying equipment, heatpumps; should not be crushed, compacted, or handled in a way that could damage any part of the refrigerant system or the insulating foam. In particular during transportation, internal transit or when loading and unloading appliances, care shall be taken to ensure that appliances’ refrigerating systems and cabinets are not damaged.  As for storage, transport and handling also the treatment of cooling and freezing appliances require precautionary protective measures due to the flammability of hydrocarbons.

### 4.1.2 Sorting

Temperature exchange equipment e.g. Refrigerators, Freezers, Equipment which automatically delivers cold products, Air conditioning equipment, Dehumidifying equipment, heatpumps; should be separated from other WEEE at logistic facilities and delivered to treatment facilities primarily specialized in the treatment of WEEE containing CFC, HCFC, HC or HFC [19]. This sorting shall only be performed by trained or adequately supervised personnel. To ensure that heat pump tumble dryers are treated correctly, as a precondition it is necessary that they are first identified and correctly sorted.

NOTE Treating WEEE without first separating appliances from other WEEE could result in emission of substances that are either harmful to the environment and present a risk of fire or explosion.

## 4.2 Technologies for the reduction of air emissions

### 4.2.1. Techniques to eliminate the release of ODS CFC / HC

- The nitrogen process [15]: The Shredding area is under air suction, CFC/Pentane is exhausted and N2 is blown in to reduce the O2 level to <4%. There is no risk of fire when the O2 is below 4%. For a shredding chamber, size of 2000x1200x1000 mm, the total amount of air exhausted would be 400m3/hour. The CFC/Pentane is then exhausted inside the unit, which applies a chemical treatment1. In this stage, the CFC/Pentane is separated from the N2 and the water. The CFC/Pentane is fed to a collecting tank for fluid CFC/Pentane. The tank is placed on a scale, which makes it possible to see how much gas is recovered and when the tank has to be changed. The N2 is recovered and send back into the Shredding area and the water is recovered and send into the drain.

- The IC process2 (Impact Catalyst)[16]: The shredding area is supplied with air continuously, so the concentration of pentane always remains far below the lower explosive limit (LEL). For this reason, inerting of the shredding area is no longer necessary. Gas purification takes place in two catalytic converters connected in series. The hydrocarbon compounds such as isobutane and pentane are oxidized in the first converter to form water and carbon dioxide. In the second reactor, the CFCs are converted into the components hydrogen, chloride (HCl), hydrogen fluoride (HF), and carbon dioxide (CO2). The HCl and HF gases are converted via washer stages into a concentrated saline solution, which must be disposed of externally.

- Cryogenic method [19]: The recycling machine automatically pumps out the harmful gasses trapped in the shredding chamber and cools them down with liquid nitrogen to minus 90 degrees Celsius. At that freezing temperature the gasses turn into liquids. The liquefied gasses are then stored in tanks.

- Recovery of CFCs from foam [18]:  The shredding takes place in a closed system to ensure the recovery of environmentally harmful CFC/HC-containing insulation and blowing agents that are released from the insulation foam during shredding.

4.3 - Technologies for the reduction of noise

A shredder mill generates noise, the permissible levels of which are often stipulated in the permit for the operation. Because the refrigerator recycling process is a close design process, the sound levels are lower than the typically open shredder plant.

It is important to consider all kinds of noise sources, including handling of the material, loading and unloading, the shredder itself, noise from fans, deflagrations etc.

## 4.4 - Technologies for the reduction of ground vibration

The planning of a refrigerator recycling plant already considers isolation elements which are adapted to their individual fields of use and conditions. Therefore, major vibrations are not to be expected from the operation of fridge shredders.

During loading and discharging of primary materials and finished goods, vibrations are detectable, but these are locally confined to the point of impact where the bulk material hits the ground plate.

## 4.5 - Technologies for the reduction of energy consumption

Technologies for the reduction of energy consumption may include measures for the reduction of peak consumption and power losses as well as the avoidance of unwanted shutdowns. Compressed air systems have to be inspected and serviced regularly to avoid leakages in pipes.

There is limited detailed monitoring/metering by shredder operators of the power they use to run the installationsMonitoring and reporting of total consumption may be undertaken for cost purposes and, for example, the rate at which the mill draws current on site may be monitored live, but it may not be recorded.

The configuration of a fridge shredder is such that the power used by the downstream separation and segregation components, and the conveyors that feed them, is typically fixed. The separation methodology is calibrated to deal with an average/ideal infeed rate and material type. There is some flexibility in the separation

processes, i.e. flexibility to increase the plant suction, adjust conveyor speed though the magnets and eddy current separators to deal with changes in the feed quality, quantity and environmental conditions at the time. The key to controlling and minimising the power usage is down to the shredding operation. The process requires that the mill chamber/box is full and the feed is consistent. The mill under these circumstances would pull a consistent current. If the feed varies greatly in density or size then the rotor will slow down during heavy loading and speed up during light loading.

In the situation of periodic heavy loading, the rotor slows, losing momentum due to the greater resistance of the material. This requires more power from the motor to increase the rotor speed and replace the lost momentum. The efficiency of the mill is also reduced when it is left running empty or the load rate falls below the optimum.

Manufacturer information suggests the power use estimates are based upon the size (infeed and mill box size) and power rating of the mill and its production capacity.

# Chapter 5 – BAT for treatment in shredders of fridges, freezers and air-conditioning units

The best available techniques in the operation of a fridge shredder plant are presented below.

## 5.1 – General BAT Conclusions

### 5.1.1 - Environmental Management Systems

BAT is to adopt an Environmental Management System appropriate to the size of the installation and the type and amount of wastes processed:

- commitment of the management, including senior management

- definition of an environmental policy that includes the continuous improvement of the installation by the management

- planning and establishing the necessary procedures, objectives and targets, in conjunction with financial planning and investment

- implementation of procedures paying particular attention to:

- Structure and responsibility

- Training, awareness and competence

- Communication

- Employee involvement

- Documentation

- Efficient process control

- Maintenance programs

- Emergency preparedness and response

- Safeguarding compliance with environmental legislation;

- checking performance and taking corrective action, paying particular attention to:

- Monitoring and measure

- Corrective and preventive action

- Maintenance of records

- independent (where practicable) internal and external auditing in order to determine whether or not the EMS conforms to planned arrangements and has been properly implemented and maintained.

- Review of the EMS and its continuing suitability, adequacy and effectiveness by senior management

- Following the development of cleaner technologies

- Consideration of the environment impacts from the decommissioning of the installation at the stage of designing a new plant, and throughout its operating life

- Application of sectorial benchmarking  on a regular basis

Examples of Environmental Management Systems are ISO 14001; EMAS; and national ant tailor made EMSs such as EfB (De) ; ESN (De); Qualicert (Fr) or similar management systems.

### 5.1.2 - Monitoring

In order to improve the overall environmental performance of waste treatment, BAT is to monitor the process parameters and the additional environmental parameters given below.

Table 8 Monitoring parameters, applicability, point of measurement and monitoring frequency

| Parameter | Applicability | Point of measurement | Monitoring Frequency |
| --- | --- | --- | --- |
| Water consumption | Measure per tonne of processed waste | Based on punchase incvoiced or meter | Continuous monitoring by meter Annual reporting in m3 |
| Energy consumption -Electricity | Generally the case but plants may also contain other electrical equipment | Based on purchase invoice and calculation or meter | Annual reporting in KWh |
| -Fuels: petrol diesel | Transport | Based on purchase invoice or meter | Batch monitoring. Annual reporting in liters |
| Air emission -Level of dust and sum of listed metals in dust | Measured in mg/Mn3 certified third party (accredited laboratory) | Generally clean air exhaust from the mill | Generally during 2 hours of production, one time per year |
| Water emission -Level of dust and sum of listed metals in dust | Measured in mg/Mm3 by certified third party (accredited laboratory) | Generally clean air exhaust from the mill | One time per year according to the receiving environment |
| Air emission -Level of dust and sum of listed metals in dust | Measured in mg/Nm3 by certified third party (accredited laboratory) | Genially clean air exhaust from the mill | Generally during 2 hours of production, one time per year |
| Water emission -Quality of water release | Material in suspension, total of listed hydrocarbons, COD, BOD5 | Point(s) of release to the environment | One or several times per year according to the receiving environment |
| Noise | Depending on location and so proximity of receptors dB | At the boundary of the facility | On a 24 hour period, one time per year according to the surrounding and neighbourhood |
| Waste -Waste received for shredding | Different types of appliances may be delivered to the facility. A separation of appliances will take place according to their content at the receipt step | Weighbridge | Monitoring per delivery. Annual reporting in tonnes |
| -Waste for disposal | Waste directly send to disposal | By Weighbridge and calculation | Monitoring per dispatch. Annual reporting in tonnes. |

(1) The electricity consumption of the whole facility is measured by the effective consumption in kWh given by the electricity suppliers (invoices). When the facility also contains other large electrical equipment there may be no separate metering but calculations can be made.

(2) Apart of that, most shredders are equipped with continuous intensity measurement but this measure is limited to the shredder engine itself and excludes conveyor belt, ventilators, etc. The registration is also time limited, for example to a 24 hours period. The intensity depends of course on the type of waste treated and the power of the shredder.

### 5.1.3 – Reception, handling and storage

Environmental performance can be improved in two principal ways: one is to control outputs and emissions at ‘end of pipe’; the other is to control the quality and type of infeed to the fridge shredder in the first place. In basic terms, what goes in will come out, as it is a simple mechanical size reduction and material separation process. In order to reduce the environmental risks in the waste treatment and to improve the waste treatment performance, BAT is to have a good and improving knowledge of waste input and a safe and sound waste input reception by using all the following techniques.

Inspection

- Fridges should be checked on arrival for damage or leaks. Those in a poor state of repair or which pose a significant risk of releasing CFC should be de-gassed immediately.

- Weighing of all refrigeration devices as they arrive

- Confirmation that sufficient storage capacity is available

Storage

- Where ODS removal will take place in two stages (degassing of refrigerant separate from foam treatment) de-gassing should be carried out prior to long term storage.

- Storage of fridges prior to de-gassing must take place in an area with an impermeable surface and sealed drainage.

- Sites that only accept pre-treated (de-gassed) fridges and do not undertake any subsequent treatment will not require impermeable surfacing.

- Units should be stored whole and upright with a maximum stacking height restriction of the equivalent of 2 upright units (approximately 3.5 metres).

- Where units have been dismantled into panels, suitable precautions must be taken to ensure the foam is not damaged e.g. removal of sharp objects, and stacking height restricted to avoid crushing of the panels.

Fire prevention

- Adequate fire prevention mechanisms should be used and access for the fire brigade should be maintained at all times.

Cleaning

- Units should be emptied of all foodstuffs, the trays and shelves removed (to aid cleaning) and cleaned prior to storage to reduce the risk of personnel being exposed to harmful or infectious material.

Storage of output materials:

- Ferrous metals and non-ferrous metals may be classified as waste or product in accordance with the respective European law , these commodities will be sold to their respective industries for use in metalworks or foundries. Likewise the plastic fractions will be shipped -as commodities to authorized facilities

- The powdery polyurethane insulating foam (polyethane granulate) is heated to around 110 oC in a close heating spiral to extract all the remaining expanding agent. The polyurethane granulate extracted from the refrigerators is a good oil binding agent.

### 5.1.4 - Reducing emissions to air

In order to prevent, or where that is not practicable, to reduce diffuse emissions from refrigerator recycling plant, BAT is to use appropriate combination of the following techniques or other techniques that would give an equivalent level of performance.

- Because refrigerator recycling plants operate as an enclosed system the techniques, which are used by general metal shredders, for reduction of dust are not appropriate. Moreover all the sorting materials are stored in a closed space, in order to eliminate the risk of release the dissolved ODS refrigerants.

- Degassing step 1: All liquids that contribute to a contamination of separated fractions during, or after, the treatment process, should be removed. Whilst all refrigerants should be separated from oil. The compressor oil with less than 0.2% by weight total halogen content  may be used for material recycling or in normal incinerators, provided national regulation permit this procedure. The compressor oil with more than 0.2% by weight total halogen content shall be treated only in thermal processes for the safe destruction of “controlled substances”. To achieve the best possible outcome refrigerant and oil should be extracted in the same step using vacuum suction. The reason for this is that the initial pressure of the system is the most effective way of removing the oil.  Proprietary degassing units are capable of removing > 99% of oil and refrigerant mix from the cooling circuit and compressor at typical temperatures of 5-10 degrees Celsius.

- The nitrogen process: This process puts the shredding area under air suction, CFC/Pentane is exhausted and N2 is blown in to reduce the O2 level to <4%. There is no risk of fire when the O2 is below 4%. For a shredding chamber size of 2000x1200x1000 mm, the total amount of air exhausted should be some 400m3/hour. The CFC/Pentane is then exhausted inside the unit, and treated chemically.. In this stage, the CFC/Pentane is separated from the N2 and water. The CFC/Pentane is pressurized in a  collecting tank for fluid CFC/Pentane. The tank is placed on a scale, which makes it possible to see how much gas is recovered and when the tank has to be changed. The N2 is recovered and send back into the Shredding area and the H2O is recovered and send into the drain.

- The IC process2 (Impact Catalyst): The shredding area is supplied with air continuously, so the concentration of pentane always remains far below the lower explosive limit (LEL). For this reason, inerting of the shredding area is no longer necessary. Gas purification takes place in two catalytic converters connected in series. The hydrocarbon compounds such as isobutane and pentane are oxidized in the first converter to form water and carbon chloride (HCl), hydrogen fluoride (HF), and carbon dioxide (CO2). The HCl and HF gases are converted via washer stages into a concentrated saline solution, which must be disposed of externally.

- Cryogenic method: The recycling machine automatically pumps out the harmful gasses trapped in the shredding chamber and cools them down with liquid nitrogen to minus 90 degrees Celsius. At that temperature the gasses turn into liquids. The liquefied gasses are stored in tanks or in gas bottles.

- Other recovery systems developed to ensure the recovery of CFC/HC-containing insulation and blowing agents that are released from the insulation foam during shredding.

An emission limit value of 150mg/m3 g CFC R11 equates to a mass loss of 75g/hr. Mass loss of ODS from the system should be proportional to the feed rate of appliances into the system. Therefore mass loss should not exceed these values:

Table 9 Maximum mass loss of ODS expressed as CFC per hour

| No. of refrigeration units per hour | Maximum mass loss of ODS expressed as CFC R11 |
| --- | --- |
| <100 | 5g per hour |
| >100 but <200 | 10g per hour |
| >200 but <300 | 15g per hour |
| >300 but <400 | 50g per hour |

See controversial point 2

The EN 50574:2012 and its Technical Specification stipulate the following limit and target values linked to the prevention of CFC/HC losses:

Step 1:

-       VFC recovery rate during tests[1] and daily business: 90 % of the expected content in the circuits (during tests expected mass of VFC according to the labels; during daily business 90 % of 126 g VFC per fridge and 54 g VHC per fridge)

-       Residual amount of VFC in oil < 0.2 % wt.

Step 2:

-       VFC/VHC recovery rate during tests[2] and daily business: 90 % of the expected amounts (Expected amounts are calculated on base of 8.5 % VFC content and 3.8 % VHC content in untreated PU foam from waste appliances in pursuance of the related foam weights fixed in the TS EN 50574)

-       Residual amount of VFC in output PU < 0.2 % wt.

-       Residual amount of adhering PU on output metal fractions < 0.3 % wt.

-       Residual amount of adhering PU on output plastic fractions < 0.5 % wt.

Emission levels for dust are as follow

Table 10 Level for dust

| Pollutant | Unit | AEL |
| --- | --- | --- |
| Dust | Mg/Nm3 | It is observed that a level <40 Mg/Nm3 can be achieved with the technologies described above in the majority of European Member States |

# Chapter 6 – Emerging techniques

## 6.1 - Thermal Oxidation

The aim of thermal oxidation is to destroy pollutants in the air that can oxidize. According to the IC process the gas purification takes place in two catalytic converters connected in series. The hydrocarbon compounds such as isobutene and pentane are oxidized in the first converter to form water and carbon dioxide. In the second reactor, the CFCs are converted into the components hydrogen chloride (HCl), hydrogen fluoride (HF), and carbon dioxide (CO2). As the process in the first reactor is exothermic, it simultaneously acts as a preheater for the second catalytic converter stage. Similarly, the thermal energy in the gases emitted from the second reactor is used to preheat the untreated gases before they enter the first reactor. This system lowers the overall energy requirements of the plant. This process layout even results in the catalytic oxidation becoming fully energy self-sufficient once the proportion of pentane refrigeration units passes a certain threshold. Furthermore – due to the reduced energy requirements – the IC process produces around ten to 20 times lower CO2 emissions (depending on the proportion of pentane refrigeration units) than conventional freeze condensation.

## 6.2 – Dissolution of VOCs

There are several technical approaches for VOCs (Volatile organic compounds) removal from shredder air systems, in principle there are four solutions which are:

- Liquidation of VOCs by forced cooling

- Adsoption of VOCs by activated carbon

- Destruction in Regenerative Thermal Oxidizers

- Dissolving VOCs in liquids

The first 3 have been proven uneconomic. However dissolution of VOCs in wash water can easily be integrated into the current filter configuration since the dust removal already includes a wet scrubber. The set up comprises a modified wet venturi scrubber and a flue gas scrubber followed by water treatment through flotation. Tests have been conducted and have resulted in collection of VOCs in the flotation agent residue, depending on the input more than 60% of VOCs were removed. Peak emissions can therefore be reduced.

Investment costs for the VOC reduction are in the order of €500,000 per installation. The increase in operating costs is negligible, whilst residues for disposal increase by 1 to 2 cubic meters per day and may be disposed of with other residues. The quality of the wash water is substantially improved.

# Chapter 7 – Concluding remarks

In order to minimize the above mentioned risks that are associated with the refrigerants, the refrigerator recycling plants should adopt the following principles:

- Fridges should be degassed prior to shredding. When oil/refrigerant mixes are extracted, the fractions should be separated as quickly as possible to allow for maximum extraction of the refrigerant from the oil. The percentage of refrigerant remaining in the oil after separation should not exceed [0.9% by weight] 2 g total halogen / kg.

- The risk of fire and explosion should be eliminated, by using monitors and control mechanisms to keep the %v/v content of the flammable and/or explosive gas (e.g. lower and the upper explosive limits of the pentane) at safe levels.

- Extraction of the foam insulation expanding agents, and their separation and collection in special containers for further processing .

- Proper storage of the usable material fractions after shredding prior to delivery to other industries.

- Control outputs and emissions at ‘end of pipe’.

The refrigerator recycling plants should provide first-grade processed materials to consumers world-wide, generating revenue for exporting countries and, as recycled materials are naturally less expensive than primary materials, offering a cost advantage to the industries that consume them. The use of recycled scrap is highly beneficial, providing appreciable energy savings and producing fewer emissions. The proper treatment for the recycling of refrigerating devices, could lead to an emission reduction of upwards of 15,000 tonnes of chlorofluorocarbon in the atmosphere in the next 10 years.

| Summary of best available techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Suggest Procedure | Report Section | Cost | Effectiveness | Feasibility |

5.1.1 Environmental Management Systems

| BAT is to adopt an Environment Management System appropriate to the size of the installation and the type and the amount of wastes processed | 5.1.1 |  |  |  |
| --- | --- | --- | --- | --- |

5.1.2 Monitoring

| Water consumption | 5.1.2 |  |  |  |
| --- | --- | --- | --- | --- |
| Energy consumption | 5.1.2 |  |  |  |
| -Electricity | 5.1.2 |  |  |  |
| - Fuels: petrol, diesel | 5.1.2 |  |  |  |
| Air emission | 5.1.2 |  |  |  |
| - Level of dust and sum of listed metals in dust | 5.1.2 |  |  |  |
| Water emission | 5.1.2 |  |  |  |
| - Quality of water release | 5.1.2 |  |  |  |
| Noise | 5.1.2 |  |  |  |
| Waste | 5.1.2 |  |  |  |
| - Waste received for shredding | 5.1.2 |  |  |  |
| - Waste for Disposal | 5.1.2 |  |  |  |

5.1.3 Reducing emissions to air

| Degassing step 1 | 5.1.3 |  |  |  |
| --- | --- | --- | --- | --- |
| The nitrogen process: | 5.1.3 |  |  |  |
| Cryogenic method | 5.1.3 |  |  |  |
| The IC process | 5.1.3 |  |  |  |

# Appendix 1 - Glossary

Best Available Techniques (BAT)

The most effective and advanced stage in the development of activities and their methods of operation which indicates the practical suitability of particular techniques for providing the basis for emission limit values and other permit conditions designed to prevent and, where that is not practicable, to reduce emissions and the impact on

the environment as a whole:

(a) ‘techniques’ includes both the technology used and the way in which the installation is designed, built, maintained, operated and decommissioned

(b) ‘available techniques’ means those developed on a scale which allows implementation in the relevant industrial sector, under economically and technically viable conditions, taking into consideration the costs and advantages, whether or not the techniques are used or produced inside the Member State in question, as long as they are reasonably accessible to the operator

(c) ‘best’ means most effective in achieving a high general level of protection of the environment as a whole. [Art. 3.10 of Dir 2010/75/CE - IED]

CFC

Chlorofluorocarbons (ODSs) means the controlled substances listed in Group I of Annex I of Regulation (EC) No 2037/2000 of the European Parliament and of the Council of 29 June 2000 on substances that deplete the ozone layer, including their isomers.

Demanufacturing:
The term ‘demanufacturing’ is used to mean the systematic recovery of secondary raw materials and the environmentally sound processing of problem substances. The demanufacture of refrigeration equipment is the mirror image of the manufacturing process, in which reproducible standards and stringent quality requirements have long been a matter of course.

GWP

Global warming potential

HCFC

Hydrochlorofluorocarbons (HCFCs) means the controlled substances listed in Group VIII of Annex I of Regulation (EC) No 2037/2000 of the European Parliament and of the Council of 29 June 2000 on substances that deplete the ozone layer, including their isomers.

IC

The product name IC stands for the terms “Impact” (for mechanical crushing using impact force) and “Catalyst” (for catalytic oxidation).

ODS

ozone-depleting substances

PCB

Polychlorinated biphenyls.  Polychlorinated biphenyls have not been used in the manufacture of new equipment since 1986, because of their toxicity and  the carcinogenic effects on humans.

Plant

The equipment, including machinery, tools, instruments, and fixtures and the buildings containing them, necessary for an industrial or manufacturing operation.

Process

The directed course of physical, chemical, biological and IT operations to ensure a desired function.

Recycling

Any recovery operation by which waste materials are reprocessed into products, material or substances, whether for the original or other purposes. It includes the reprocessing of organic material but does not include energy recovery and the reprocessing into materials that are to be used as fuel or for backfilling operations. [Art. 3.17 of Dir 2008/98/CE - WFD]

Removal

manual, mechanical, chemical or metallurgic handling with the result that hazardous substances, mixtures and components are contained in an identifiable stream or are an identifiable part of a stream within the treatment process. A substance, mixture or component is identifiable if it can be monitored to verify environmentally safe treatment;

VOCs

Volatile organic compounds

VFC

volatile fluorocarbon

VHC

Volatile hydrocarbon

WEEE

waste electrical and electronic equipment

# References

[1] http://www.arctick.org/pdf/Instruction_Booklet%20final.pdf

[2] http://www.epa.gov/ozone/title6/608/disposal/household.html

[3] file:///C:/Users/eco/Downloads/BHS_RS_EN-US_2012-02.pdf

[4]http://www.oppd.com/prodconsump10g/groups/web/documents/webcontent/22_006087.pdf

[5] http://climatizzazione.mitsubishielectric.it/en/informazioni-utili/componenti-principali.php

[6] http://www.refrigerants.com/

[7] http://www.draeger.com/sites/assets/PublishingImages/Segments/ES/Oil-Gas-Industry/Plant-Safety-Operations/explosion_protection_br_9046262_en.pdf

[8] http://www.eldan-recycling.com/content/refrigerator-recycling

[9.1] http://www.ipcc.ch/publications_and_data/ar4/wg1/en/ch2s2-10-2.html

[9.2]  http://www.srcosmos.gr/srcosmos/showpub.aspx?aa=13135

[10] http://www.environmental-expert.com/companies/keyword-refrigerator-recycling-plants-4069/location-europe/page-8

[11] http://www.paseppe.gr/library/Downloads/docs/pae_13_11_08.pdf

[12] Guidance on the Recovery and Disposal of Controlled Substances Contained in Refrigerators and Freezers (2002)

[13] http://www.erdwich.com/

[14] http://www.untha.com/de

[15] http://www.varygroup.com/

[14] http://media.ufhrecycling.at/files/ufh/837.pdf

[15] http://www.industria.gr/

[16] http://www.bhs-sonthofen.de/en.html

[17] http://www.gereports.com/post/74545071720/appetite-for-destruction-giant-fridge-shredder-hits

[18] http://www.era-gmbh.at/en/collection-and-recovery/recovery/recovery-of-weee.html

[19] EN 50574:2012, Collection, logistics & treatment requirements for end-of-life household appliances containing volatile fluorocarbons or volatile hydrocarbons (2012)

[20] http://www.beuth.de/de/technische-regel/ral-gz-728/180021026

[21] http://www.oeko.de/publikationen/p-details/untersuchung-des-ozonzerstoerungspotentials-bei-der-durchfuehrung-des-kuehlgeraeterecyclings-mittels/

[22] http.//www.seg-online.de

[23] TS EN 50574:2014 Collection, logistics & treatment requirements for end-of-life household appliances containing volatile fluorocarbons or volatile hydrocarbons - Part 2: specification for de-pollution

[24] REGULATION (EC) No 1005/2009, on substances that deplete the ozone layer.

[25] WEEELABEX project Treatment. http://www.weee-forum.org/weeelabexproject

Additional information have been collected from

GUANGZHOU 3E MACHINERY CO LTD

Hunan VARY Tech.,Co.,Ltd

UNTHA Recycling Technologies

ELDAN Recycling

ADELMANN Umwelt GmbH

Temperature exchange equipment e.g. Refrigerators, Freezers, Equipment which automatically delivers cold products, Air conditioning equipment, Dehumidifying equipment, heatpumps; i.e. equipment containing gases that are ozone depleting or have a global warming potential (GWP) above 15.

Directive 2012/19/EU on waste electrical and electronic equipment (WEEE), Annex VII, Section 1 header and  indent 9: 1. As a minimum the following substances, mixtures and components have to be removed from any separately collected WEEE: - chlorofluorocarbons (CFC), hydrochlorofluorocarbons (HCFC) or hydrofluorocarbons (HFC), hydrocarbons (HC),.

Directive 2012/19/EU on waste electrical and electronic equipment (WEEE), Annex VII, Section 2, indent 2: equipment containing gases that are ozone depleting or have a global warming potential (GWP) above 15, such as those contained in foams and refrigeration circuits: the gases must be properly extracted and properly treated. Ozone-depleting gases must be treated in accordance with Regulation (EC) No 1005/2009,

Direct-reduced iron (DRI), also called sponge iron, is produced by the direct reduction of iron ore (in the form of lumps, pellets or fines) by a reducing gas produced from natural gas or coal. DRI is an alternative to ferrous scrap as a feedstock for electric arc furnaces (EAFs)

Council Regulation (EU) No 333/2011 of 31 March 2011 establishing criteria determining when certain types of scrap metal cease to be waste under Directive 2008/98/EC of the European Parliament and of the Council (scope: iron, steel and aluminium scrap, including aluminium alloy scrap)

Commission Regulation (EU) No 715/2013 of 25 July 2013 establishing criteria determining when copper scrap ceases to be waste under Directive 2008/98/EC of the European Parliament and of the Council

Temperature exchange equipment e.g. Refrigerators, Freezers, Equipment which automatically delivers cold products, Air conditioning equipment, Dehumidifying equipment, heatpumps; i.e. equipment containing gases that are ozone depleting or have a global warming potential (GWP) above 15.