Physico-chemical treatment of wastes containing POPs

(introduction for section 5.8.1)

The following disposal operations, as provided for in Annexes IV A and IV B of the Basel Convention, should be permitted for the purpose of destruction and irreversible transformation of the POP content in wastes when applied in such a way as to ensure that the remaining wastes and releases do not exhibit the characteristics of POPs:

(a)	D9 Physico-chemical treatment;

(b)	D10 Incineration on land;

(c)	R1 Use as a fuel (other than in direct incineration) or other means to generate energy;

(d)	R4 Recycling/reclamation of metals and metal compounds, but restricted to activities of primary and secondary metallurgy described in (k) below.

POPs that are isolated from the waste during pre-treatment should subsequently be disposed of in accordance with operations D9 and D10.

Evaluation of the performance of technologies for treatment of POPs waste

For evaluation of performance of technologies used for POPs waste disposal evaluation of their Destruction Efficience (DE) and Destruction and Removal Efficiency (DRE) were introduced.

The destruction efficiency  (DE) is the percentage of originating POPs destroyed or irreversibly transformed by a particular method or technology. The destruction removal efficiency  (DRE) only considers emissions to air and is the percentage of original POPs irreversibly transformed and removed from the gaseous emissions.

(a)	Both DE and DRE are a function of the initial POP content and do not cover unintentional or production of new POPs during destruction or irreversible transformation;

(b)	DE is an important criterion to assess technologies performance for destruction and irreversible transformation, but can be difficult to measure in a reproducible and comparable manner . The following provisional definition for levels of destruction and irreversible transformation, based upon absolute levels (i.e., waste output streams of treatment processes) should be applied:

(a)	Atmospheric emissions:

(i)	PCDDs and PCDFs: 0.1 ng TEQ/Nm3;

(ii)	All other POPs: pertinent national legislation and international rules, standards and guidelines, examples of pertinent national legislation can be found in annex II;

(b)	Aqueous releases: pertinent national legislation and international rules, standards and guidelines, examples of pertinent national legislation can be found in annex II;

(c)	Solid residues: POP contents should be below the low POP contents defined in section A of this chapter. However, if the POP content of unintentionally produced PCDDs/PCDFs is above the low POP content defined in section A of Basel Technical Guidelines .

Technologies for destruction and irreversible transformation of the POP content in wastes

The table xx below and following subsections describe commercially available operations for the environmentally sound destruction and irreversible transformation of the POP content in wastes that are capable to achieve the levels of destruction and irreversible transformationTable 5.202 provides an overview of these operations. It should be noted that pertinent national legislation apply for these operations and that they should be used according to the BAT and BEP standards developed by the Stockholm Convention, as presented in the Guidelines on best available techniques and provisional guidance on best environmental practices relevant to Article 5 and Annex C of the Stockholm Convention on Persistent Organic Pollutants .

Emerging technologies are not described in this sub-section. For assessing the performance of the operations in the subsections below, a minimum DE of 99.999 %, with 99.9999 % of DRE as a supplement requirement where applicable, provides practical benchmark parameters for assessing disposal technology performance. Higher demonstrated DEs may be preferred on a case-by-case basis. DE and DRE should be used in conjunction to demonstrate a level of destruction and irreversible transformation; as neither DE nor DRE take into account the potential for transformation of the original to an unintentionally produced POP, such releases should be considered in the choice of an operation.  Releases data of PCDD/Fs and dioxin-like PCBs (DL PCBs) from physico-chemical treatment of POPs waste technologies is summarized in Table 5.203.

Table 5.202: Overview of technologies for the destruction and irreversible transformation of POPs in waste .

| Technology | POPs |  |  |
| --- | --- | --- | --- |
|  | Pesticide POPs | PCB | PCDDs/PCDFs |
| - Alkali metal reduction | Yes, for certain pesticides: chlordane, HCH | Yes | ND* |
| - Advanced Solid waste incineration (ASWI) | ND | ND | ND |
| - Base catalyzed decomposition (BCD) | Yes, for certain pesticides: chlordane, HCH DDT | Yes | Yes |
| - Catalytic hydrodechlorination (CHD) | ND | Yes | Yes |
| - Cement kiln co-incineration | Yes for all pesticides | Yes | Yes |
| - Gas phase chemical reduction (GPCR) | Yes, for certain pesticides: DDT, HCB | Yes | Yes |
| - Hazardous waste incineration | Yes for all pesticides | Yes | Yes |
| - Plasma Arc | Yes, for most pesticides including chlordane, chlordecone, DDT, endosulfan, heptaclor | Yes | ND |
| - Plasma melting decomposition method (PMD) | ND | Yes | ND |
| - Supercritical water oxidation (SCWO) and subcritical water oxidation | Yes, for certain pesticides: chlordane and DDT | Yes | Yes for PCDDs |
| - Thermal and metallurgical production of metals | ND | ND | Yes |

*ND stands for “not determined” and indicates that information is not available to confirm the use of the technology for certain POPs.

Waste incineration, plasma technologies, cement kilns and metallurgical techniques are out of scope of this BREF. All of them are also listed as unintentionally produced POPs in Annex C to Stockholm Convention

Table 5.203: Comparison of concentrations of PCDD/Fs and dioxin-like PCBs in outputs from the individual technologies for POPs-containing waste decontamination

| Technology | Air releases (ng TEQ/m3) | Water discharge (ng TEQ/l) | By-product waste residues | Treated material (ng TEQ/g) | Processing matrix (ng TEQ/g) | Source |
| --- | --- | --- | --- | --- | --- | --- |
| PCDD/Fs and dioxin-like PCBs |  |  |  |  |  |  |
| Alkali metal reduction | <0.002 | 0.00005 – 0.0001 | NA | 0.0018 | NA |  |
| Catalytic-hydro dechlorination | 0.0001 | NA | NA | NA | 0.00001 - 0.0001 |  |
| Photochemical dechlorination and catalytic dechlorination reaction | 0.00007 | NA | NA | NA | NA |  |
| Supercritical Water Oxidation | 0.001 – 0.002 | 0.0000005 | NA | NA | NA |  |
| Subcritical Water Oxidation | 0.00009 | NA | NA | NA | NA |  |
| PCDD/Fs only |  |  |  |  |  |  |
| BCD (Sydney, Olympic site) | 0.0119 - 0.05 | NA | NA | NA | NA |  |
| BCD (Japan) | <0.01 | NA | NA | NA | NA |  |
| BCD (Spolana Neratovice - pilot) | 0.013 - 0.031 | NA | NA | NA | <0.016 |  |
| BCD (Spolana Neratovice - full) | 0.0017 - 0.0424 | 0 - 1.4 | NA | 0 - 0.98 | NA |  |
| Gas Phase Chemical Reduction | 0,0000028 - 0.00027 (<0.016) | 0.00000061 - 0.00084 | NA | NA | no processing matrix |  |

Alkali Metal Reduction (Sodium Reduction)

Principle of operation

Reaction of metallic alkali with chlorine atoms contained in the chlorinated compounds

Process description

Alkali metal reduction involves the treatment of wastes with dispersed alkali metal. Alkali metals react with chlorine in halogenated waste to produce salts and non-halogenated waste. Typically, the process operates at atmospheric pressure and temperatures between 60 °C and 180 °C . Treatment can take place either in situ (e.g., PCB-contaminated transformers) or ex situ in a reaction vessel. There are several variations of this process . Although potassium and potassium-sodium alloy have been used, metallic sodium is the most commonly used reducing agent.

The process must avoid the formation of a polymer (which occurs in one or two of the technologies identified) or must take the formation of such this solid into account and introduce a separation step to yield the pure reusable oil.

Ex-situ treatment of PCBs can be performed, however, following solvent extraction of PCBs. Treatment of whole capacitors and transformers could be carried out following size reduction through shearing. Pre-treatment should include dewatering by phase separation, evaporation, or other method (UNIDO, 1987) to avoid explosive reactions with metallic sodium. Equipment should be washed with organic solvents. Similarly, the POPs which are solid or in the adsorbed state would need to be dissolved to the required concentration or extracted from matrices .

Immediate energy requirements are expected to be relatively low owing to the low operating temperatures associated with the sodium reduction process.

The process is available in transportable and fixed configurations . Mobile facilities are capable of treating 15,000 litres per day of PCBs transformer oil .

This process has been used commercially for about 25 years and is still in use today.

Performance levels

Destruction efficiency (DE) values of greater than 99.999 % and destruction removal efficiency (DRE) values of 99.9999 % have been reported for chlordane, HCH and PCBs . Apart from European Union, the sodium reduction process has also been demonstrated to meet regulatory criteria in Australia, Canada, Japan, South Africa and the United States of America for PCB transformer oil treatment, i.e., less than 2 mg/kg in solid and liquid residues .

Levels of PCDD/Fs and DL PCBs in different outlets are presented in Table 5.203.

Feed and output streams

Waste Input: Sodium reduction has been demonstrated with PCB-contaminated oils containing concentrations up to 10,000 mg/kg. Some vendors have claimed that this process is capable of treating whole capacitors and transformers . It is also reported that Alkali metal reduction is applicable to chlordane and HCH as waste pesticides .

Material requirements: Sodium is required to operate this process .

Largest number of Alkali Metal Reduction facilities is based in Japan. The plants operated in 2006 were treating oils contaminated with polychlorinated biphenyls (PCB). A number of plants treated PCB with concentrations in the range of 100 parts per million, while some plants were treating concentrations up to 10 per cent. In all plants, the exhaust gas was treated with activated carbon .

Outputs: organic compounds (oil which may be reused) and salt.

Air emissions include nitrogen and hydrogen gas. Emissions of organic compounds are expected to be relatively minor . It has been noted, however, that PCDDs and PCDFs can be formed from chlorophenols under alkaline conditions at temperatures as low as 150°C , however low levels of PCDD/Fs were measured in full scale operation units (see Table 5.203). Residues produced during the process include sodium chloride, sodium hydroxide, polyphenyls and water. In some variations, a solidified polymer is also formed .

After the reaction, the by-products can be separated out from the oil through a combination of filtration and centrifugation. The decontaminated oil can be reused, the sodium chloride can either be reused or disposed of in a landfill and the solidified polymer can be disposed of in a landfill . No or minimal post-treatment is required. Depending on technology such a method can include off-gas treatment and neutralization or conservation of residuals. The excess of sodium, if not neutralized, may need to be recovered. Minor quantities of volatile organics in emissions can be captured by activated carbon .

Reference literature

Base catalyzed decomposition (BCD)

Principle of operation

The BCD process involves treatment of wastes in the presence of a reagent mixture consisting of hydrogen-donor oil, alkali metal hydroxide and a proprietary catalyst.

Process description

When the mixture is heated to above 300 °C, the reagent produces highly reactive atomic hydrogen. The atomic hydrogen reacts with the waste to remove constituents that confer the toxicity to compounds.

Soils may be treated directly, however most often different types of soil pre-treatment is necessary: a) Larger particles may need to be removed by sifting and crushed to reduce their size;  b) pH and moisture content may need to be adjusted, and c) Indirect Thermal Desorption (ITD) is also used in conjunction with BCD to remove POPs from soils prior to treatment. In these situations, the soil is pre-mixed with sodium bicarbonate prior to being fed into the thermal desorption unit. Water will need to be evaporated from aqueous media, including wet sludge, prior to treatment. Capacitors can be treated following size reduction through shredding. If volatile solvents are present, such as occurs with pesticides, they should be removed by distillation prior to treatment .

The equipment associated with this process is readily available .

Modular, transportable and fixed plants have been built .

Since the BCD process involves stripping chlorine from the waste compound, the treatment process may result in an increased concentration of lower-chlorinated species. This can be of potential concern in the treatment of PCDDs and PCDFs, where the lower chlorinated congeners are more toxic than the higher-chlorinated congeners. It is therefore important that the process be appropriately monitored to ensure that the reaction continues to completion.

BCD has been used at two commercial operations within Australia, with one still operating. Another commercial system has been operating in Mexico since 1999. In addition, BCD systems have been used for short-term projects in Australia, Czech Republic, Spain and the United States of America. It was used for example to clean up PCDD/Fs and chlorinated pesticides contamination in Spolana Neratovice, Czech Republic .

Performance levels

DEs of 99.99–99.9999 % have been reported for DDT, PCBs, PCDDs and PCDFs . DEs of greater than 99.999 % and DREs of greater than 99.9999 % have also been reported for chlordane and HCH . It has also been reported that reduction of chlorinated organics to less than 2 mg/kg is achievable .

Data on emissions from base catalysed decomposition for two plants operating in Japan, one plant operating in Australia and one plant operating in the Czech Republic were provided to the Stockholm Convention expert group on BAT/BEP , and are summarized in Table 5.203. The first three treated  PCB  at varying levels up to 10 per cent. It was noted that a former plant which treated soil contaminated with a range of persistent organic pollutants at the Sydney Olympic site in Australia had been decommissioned in 2003.

Measurements carried out during operation of the BCD technology in Spolana Neratovice described below enable calculation of the emission factor for PCDD/Fs concerning this technology. For the calculation, were used the following data: volume of waste gas released from APS (2,560 m3/hour, in total 15,400,000 m3), and the average value of dioxin concentration . The emission factor calculated on this basis for dioxins for the BCD technology is 0.0168 to 0.0173 µg (TEQ)/ton of waste. In the case of hazardous waste incineration, emission factors 4.57 - 90, and 0.5 - 350 µg (TEQ)/ton of incinerated waste are stated for the Czech Republic and for Poland, respectively .

Feed and output streams

BCD should be applicable to other POPs in addition to the ones listed above . BCD should be capable of treating wastes with a high POP concentration, with demonstrated applicability to wastes with a PCB content of above 30 % by weight .

Air emissions are expected to be relatively minor. The potential to form PCDDs and PCDFs during the BCD process is relatively low. However, it has been noted that PCDDs can be formed from chlorophenols under alkaline conditions at temperatures as low as 150 °C  . In full scale operation plant treating materials contaminated by PCDD/Fs and chlorinated pesticides has shown levels of PCDD/Fs within the range of < 0.0006 - 0.079 ng TEQ/m3. For Σ OCP the observed emissions were  within the range of 0.000024 – 0,0109 mg/m3 . Other residues produced during the BCD reaction include sludge containing primarily water, salt, unused hydrogen-donor oil and carbon residue.

Depending on the type of hydrogen-donor oil used, the slurry residue may be treated in different ways. If more refined oils are used, these may be removed from the sludge by gravity or centrifuge separation. The oils can then be reused and the remaining sludge can be further treated for use as a neutralizing agent or disposed of in a landfill . In addition, BCD plants are equipped with activated carbon traps to minimize releases of volatile organics in gaseous emissions.

Material requirements:

a) Hydrogen-donor oil

b) Alkali or alkaline earth metal carbonate, bicarbonate or hydroxide, such as sodium bicarbonate. The amount of alkali required is dependent on the concentration of the halogenated contaminant contained in the medium . Amounts range from 1 % to about 20 % by weight of the contaminated medium; and

c) Proprietary catalyst amounting to 1 % by volume of the hydrogen donor oil.

BCD operation within remediation of PCDD/Fs and OCPs contaminated site in Spolana, Neratovice (Czech Republic)

In total, over 40 thousand tons of soils and materials from building demolition were subjected to the decontamination process. The actual BCD reactor processed 150 tons of material, because it was preceded by indirect thermal desorption (ITD) and annealing furnace (MPF). The actual BCD reactor processed a process oil, where also POPs were concentrated. Information on inputs and outputs, i.e., POPs concentrations in oil after processing in the reactor, is available only from the pilot operation of the unit .

For full scal facility in the Czech Republic: In the mixed waste water samples, monitored in the interval of one to two weeks, there were found PCDD/Fs concentrations between 0 and 1.4 ng TEQ/l (average 0.19; median 0.1; standard deviation 0.28). In the mixed samples of soil after decontamination, the PCDD/Fs concentrations were from 0 to 3.3 ng TEQ/g (average 0.07; median 0.01; standard deviation 0.26) and OCPs concentrations from < 0.1 to 7.9 mg/kg (average 0.81; median 0.41; standard deviation 1.02). Soils with concentrations exceeding the target decontamination limits of 2004 (1 ng TEQ/g PCDD/Fs, and 30 mg/kg OCPs) were returned back to repeated decontamination .

Levels of OCPs in waste inputs as well as in output material are shown in Table 5.204. The same for PCDD/Fs is summarized in Table 5.205.

Table 5.204: Hexachlorobenzene and lindane decomposition by the BCD process during pilot operation in Spolana Neratovice. Source .

| Material | Inlet mg/kg |  | Outlet Oil Matrix mg/kg |  |
| --- | --- | --- | --- | --- |
|  | HCB | Lindane | HCB | Lindane |
| Chemical waste | 29,000 | 1,500 | < 1.0 | < 1.0 |
| Chemical waste | 200,000 | 900 | < 2.0 | < 2.0 |
| Chemical waste | 550,000 | 1,000 | < 2.0 | < 2.0 |
| Chemical waste | 270,000 | 1,000 | < 2.0 | < 2.0 |
| Chemical waste | 160,000 | 1,000 | < 2.0 | < 2.0 |
| Dust | 7,600 | 7 | < 2.0 | < 2.0 |
| Chemical waste | 1,598 | 19,000 | < 2.0 | < 2.0 |
| Concentrate Aqueous | 630 | < 2.0 | < 2.0 | < 2.0 |
| Concentrate Organic | 11,000 | < 2.0 | < 2.0 | < 2.0 |

Table 5.205: Dioxin concentrations at inputs and outputs of the ITD and BCD technologies, according to the study published in Poland. Source .

| Levels of PCDD/PCDF [TEQ ng/g] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix | Technology |  |  |  |
|  | ITD/BCD |  | BCD |  |
|  | inlet | outlet | inlet | outlet (oil) |
| concrete | 4780 | 0.066 |  |  |
| soil | 45 | 0.0030 |  |  |
| concentrate aqueous from ITD |  |  | 96170 | 0 |
| waste chemicals |  |  | 209 | 0 |

Reference literature

Catalytic hydrodechlorination (CHD)

Principle of operation

Hydrogen reacts with chlorinated organic compounds or non-chlorinated organic contaminants, such as PAHs, at high temperatures

Process description

CHD involves the treatment of wastes with hydrogen gas and palladium on carbon (Pd/C) catalyst dispersed in paraffin oil. Hydrogen reacts with chlorine in halogenated waste to produce hydrogen chloride (HCl) and non-halogenated waste. In the case of PCBs, biphenyl is the main product. The process operates at atmospheric pressure and temperatures between 180 °C and 260 °C .

PCBs and PCDDs/PCDFs must be extracted using solvents or isolated by vaporization. Substances with low boiling points such as water or alcohols should be removed by distillation prior to treatment.

Biphenyl, the main product, is separated out from the reaction solvent by distillation after the reaction, and the catalyst and reaction solvent are reused for the next reaction .

Energy requirements are expected to be relatively low owing to the low operating temperatures associated with the CHD process.

The CHD process requires the same number of atoms of hydrogen as those of chlorine in the PCBs, and also 0.5 % by weight of catalyst.

CHD is available in fixed and transportable configurations depending on the volume of PCBs to be treated. In Japan, a plant which is capable of treating 2 Mg PCB per day using the CHD process was constructed and is in operation. Plants are run in Canton, United States of America and in Young, Australia. In Japan, a commercial-scale plant was constructed at the Japan Environmental Storage & Safety Corporation (JESCO) Osaka facility in 2006 and PCB extracted from transformers and capacitors are treated by the CHD process .

There are many reports about PCB dechlorination by using CHD. Generally, Pd/C catalyst shows the largest degradation rate compared to the other supported metal catalysts. Reaction temperature can be increased to 260 °C when paraffin oil is used as reaction solvent .

Performance levels

DEs of 99.98–99.9999 % have been reported for PCBs. It has also been reported that a reduction of the PCB content to less than 0.5 mg/kg is achievable .

Levels of PCDD/Fs and DL PCBs in different outlets are presented in Table 5.203.

Feed and output streams

Waste input IN: CHD has been demonstrated with PCBs removed from used capacitors. PCDDs and PCDFs contained in PCBs as impurities have also been dechlorinated. A vendor has also claimed that chlorinated wastes in liquid state or dissolved in solvents can be treated by CHD.

Output waste OUT: No emissions would occur during the dechlorination reaction because it takes place in the closed hydrogen circulation system. HCl is not discharged from the reaction because it is collected with water as hydrochloric acid within the circulation system. Biphenyl isolated after the reaction by distillation does not contain any toxic materials .

Reference literature

Gas phase chemical reduction (GPCR)

Principle of operation

The GPCR process involves the thermochemical reduction of organic compounds.

Process description

At temperatures greater than 850 °C and at low pressures, hydrogen reacts with chlorinated organic compounds to yield primarily methane, hydrogen chloride (if the waste is chlorinated), and minor amounts of low molecular weight hydrocarbons (benzene and ethylene). The hydrochloric acid is neutralized through the addition of caustic soda during the initial cooling of the process gas, or can be taken off in acid form for reuse. The GPCR technology can be broken down into three basic unit operations: the front-end system (where the contaminants are rendered into a suitable form for destruction in the reactor), the reactor (which reduces the contaminants, now in gas phase, using hydrogen and steam), and the gas scrubbing and compression system .

Contaminants must be in a gaseous form in order to be reduced in the GPCR reactor. While liquid wastes can be preheated and injected directly into the reactor on a continuous basis, contaminants on solids must first be volatilized from the solid. Depending on the waste type, one of the following three pre-treatment units is used to volatilize wastes prior to treatment in the GPCR reactor:

(a)	Thermal reduction batch processor (TRBP) for bulk solids, including those in drums;

(b)	Toroidal bed reactor for contaminated soils and sediments, but also adapted for liquids;

(c)	Liquid waste pre-heater system (LWPS) for liquids.

In addition, other pre-processing is required for large capacitors and building rubble. Large capacitors are punctured and drained, while rubble and concrete must be reduced in size to less than one square meter.

Gases leaving the reactor are scrubbed to remove water, heat, acid and carbon dioxide.  Scrubber residue and particulate will require disposal off site.  Solid residues generated from solid waste inputs should be suitable for disposal in a landfill.

Methane produced during the process can provide much of the fuel needs.  It has been reported that electricity requirements range from 96 kWh per ton of soil treated to around 900 kWh per ton of pure organic contaminants treated.

There is a need for hydrogen supplies, at least during start-up. It has been reported that methane produced during the GPCR process can be used to form enough hydrogen to operate the process thereafter.  The hydrogen production unit was plagued, however, by reliability problems in the past.  Other material requirements include caustic for the acid scrubber.

GPCR is available in fixed and transportable configurations.

GPCR treats sewage sludge by converting the waste into clean water and a clean hydrogen enriched methane gas while chemically destroying all pathogens and pharmaceuticals and recovering phosphorous. There are no fugitive methane emissions.

GPCR process capacity is dependent on the capacity of the three pre treatment units, as specified below:

(a)	TRBP has a capacity of up to 100 tons of solids per month or up to four litres per minute of liquids. Two TRBPs can be used in parallel to double capacity;

(b)	Toroidal bed reactor has a capacity of up to 5,000 tons of soils and sediments per month, although this pre-treatment unit is still in the development stage; and

(c)	LWPS has a capacity of three litres per minute .

Commercial-scale GPCR plants have operated in Canada and Australia. The GPCR plant in Australia operated for more than five years until 2000.  In the United States, it is planned to build a GPCR synthetic diesel plant with a 200 Mg per day capacity in Fauquier County, Virginia.

Performance

DEs of 99.9999 % have been reported for DDT, HCB, PCBs, PCDDs and PCDFs.

A Danish review  noted that emissions of PCDD/PCDF from the gas phase chemical reduction process to all media were lower than those from the base catalysed decomposition process. Levels of PCDD/Fs and DL PCBs in different outlets are presented in Table 5.203.

Feed and output streams

GPCR successfully treated different types of organic wastes including chemical warfare agents, explosives, pesticides, brominated fire retardants, CFC refrigerants, HCB, and dioxins. The technology is proven suitable for the destruction of organic wastes in all matrices including soil, sediment, sludge, high-strength oils, tar, watery wastes, wood wastes, and bulk solids such as electrical equipment, equipment casings, and drums .

In addition to hydrogen chloride and methane, low molecular weight hydrocarbons may be emitted. Residues from the GPCR process include used liquor and water. Solid residues will also be generated from solid waste inputs.  Since the GPCR process takes place in a reducing atmosphere, the possibility of PCDD and PCDF formation is considered limited.  No ash is produced .

Reference literature

Supercritical water oxidation (SCWO)

Principle of operation

A high temperature and pressure technique that uses the properties of supercritical water in the destruction of POPs

Process description

SCWO and subcritical water oxidation treat wastes in an enclosed system using an oxidant (such as oxygen, hydrogen peroxide, nitrite, nitrate, etc.) Supercritical water oxidation completely oxidizes and decomposes toxic organic substances like PCB using the characteristics of water (supercritical water) exceeding critical conditions (374 °C, 22MPa). Supercritical water has good characteristics as an oxidation decomposition reaction catalyst for freely dissolving organic materiál and oxygen. Typical reaction conditions are: temperature 400 - 650 °C, pressure 25MPa, reaction time 1 - 5 mins. When this technique is applied to treatment of waste pressure-sensitive paper, both the PCB and paper component are simultaneously and completely decomposed. With this technique, the carbon in waste pressure-sensitive paper is converted to carbon dioxide, and the hydrogen is converted to water. Chlorine is ionized and discharged as HCl. If the PCB concentration is high, the concentration of the produced HCl increases, so it is neutralized by adding alkali .

Concentrated wastes may have to be diluted prior to treatment in order to reduce the organic content to below 20 %. In the case of subcritical water oxidation, dilution of wastes is not necessary. If solids are present, they must be reduced to less than 200 µm in diameter. Other processing options include addition of fuel to low concentration wastes, co-processing of dilute and concentrated wastes, partial sludge dewatering and others.

Energy requirements are expected to be relatively high because of the combinations of high temperatures and pressures. It has been claimed, however, that as long as relatively high hydrocarbon content is present in the feed, no energy input is required to heat the feed to supercritical temperatures.

The SCWO and subcritical water oxidation reaction vessel must be constructed of materials capable of resisting corrosion caused by halogen ions.  Material corrosion can be severe at the temperatures and pressures used in the SCWO and subcritical water oxidation process. In the past, the use of titanium alloys has been proposed to tackle this problem. Current vendors claim to have overcome this problem through the use of advanced materials and engineering designs.

The SCWO and subcritical water oxidation units are currently used in a fixed configuration, but are thought to be transportable .

Current SCWO demonstration units are capable of treating 500 kg/h, while full-scale units can be designed to treat 2,700 kg/h.  A full-scale commercial plant with a capacity of 2 tons per day was installed in 2005 and is in operation in Japan . In addition, the SCWO process has been approved for full-scale development and use in the chemical weapon destruction programme of the United States of America.

Earlier designs were plagued by reliability, corrosion and plugging problems. Current vendors claim to have addressed these problems through the use of special reactor designs and corrosion-resistant materials.

Performance

DEs of greater than 99.999 % and DREs of greater than 99.9999 % have been reported for chlordane, DDT and PCBs for SCWO . DEs of greater than 99.999999 and DREs of greater than 99.9999999 % have been reported for subcritical water oxidation . DREs as high as 99.9999 % have also been demonstrated for PCDDs in bench scale tests.  SCWO has been shown effective in the treatment of toxic chlorinated chemicals such as polychlorinated biphenyls, pesticides, flame retardants . Overall, SCWO usually as >99.99 % destruction efficiency for wide range of organic compounds .

Levels of PCDD/Fs and DL PCBs in different outlets are presented in Table 5.203.

Feed and output streams

Waste IN: wastes containing organic compounds and toxic wastes. SCWO and subcritical water oxidation are applicable to all POPs . Applicable waste types include aqueous wastes, oils, solvents and solids with a diameter of under 200 µm. The organic content of the waste is limited to below 20 % by weight.

Waste OUT: carbon dioxide; hydrogen to water; the chlorine atoms derived from chlorinated organic compounds to chloride ions; nitro-compounds to nitrates; sulphur to sulphates; and phosphorus to phosphate.

During laboratory-scale PCB destruction, it was shown that the SCWO technology has the potential to form high concentrations of PCDFs (in the % range) during PCB degradation even at temperatures of practical operation . It has been reported that emissions contain no oxides of nitrogen or acid gases such as hydrogen chloride or oxides of sulphur and that process residues consist of water and solids if the waste contains inorganic salts or organic compounds with halogens, sulphur or phosphorus.  Limited information has been reported regarding potential concentrations of undestroyed chemicals.  The process is designed so that emissions and residues can be captured for reprocessing if needed.  The SCWO process is typically operated under temperatures of 600-650 °C and pressures of 22-25MPa  with reactor residence times under one minute for complete destruction. Under these conditions, PCDD/PCDF, NOx and other toxic by-products are not formed in SCWO .

Reference literature

Dechlorination with KPEG

Principle of operation

Reaction of potassium hydroxide (KOH) and polyethylene glycol (PEG) with chlorine atoms contained in the chlorinated compounds

Process description

Chlorines from PCBs are removed by polyethylene glycol (PEG) acting as a nucleophile in reaction with PCBs under the basic condition. This process has proven to be practical and highly cost-effective for PCBs treatment directly in transformer oil and the recovery of the treated oil. Furthermore, the process avoids the use of potentially dangerous alkali metals and alkali metal oxides being generally employed as catalysts.

As the process is carried out at low temperatures (140–160 °C) the formation of furans and dioxins by thermal decomposition of PCB is less likely to occur. The gases released are carbon dioxide and water vapour, which are retained by a filter of activated carbon.

The waste generated by the treatment forms a 'sludge' product of the combination of chlorine molecules of PCBs extracted with the alkaline compounds. This 'mud' (alkali chloride) precipitates due to the insolubility and weight, by separating the cleared oil.

Dechlorination treatment can reach less than 2 ppm (ASTM D4059 method) .

Feed and output streams

Waste Input: Oils contaminated with PCB (less than 5000 ppm)

Output: mineral oil, which may be reused after a regeneration treatment, and sludge (KCl)

Reference literature

Photochemical dechlorination (PCD) and catalytic dechlorination (CD) reaction

Only one small plant was commercially operating and it was located in Japan. It was noted that it had a very small capacity of 4.6 kg/2 days of high concentration PCB . Levels of PCDD/Fs and DL PCBs in different outlets are presented in Table 5.203.

NOTE: Original text and Table 5.202: This section concentrates on the physico-chemical treatments and they are summarised below in Table 5.202, related to these very specific types of waste (e.g. waste containing PCBs, dioxins and furans).

Table converted into text and updated by using new documents approved by Basel Convention COP12.

From ex-Section 4.3.3.1 "Treatment of oil contaminated with PCB"

Applicability

This technique is applied to oil transformers with PCB contents of between 25 ppm and 2000 ppm. The process can be applied to the following:

 Dehalogenation and recovery of bulking ex-transformer mineral oils contaminated with PCBs.

 Cleaning and recovery of operating transformers containing mineral oils contaminated with PCBs. This step is performed by means of recirculation of the dehalogenated mineral oil and it permits the extraction of the residual PCBs absorbed in the different components of the transformers (e.g. paper and wood).

Economics

Summarized in following table:

| Technology | Vendor | PCB Oils | Soils | Capacitors | Transformers |
| --- | --- | --- | --- | --- | --- |
| Ball Milling 1 | EDL | $300/ton | $250/ton | $300/ton | $300/ton |
| Base Catalyzed Decomposition 2 | Multiple | $0.7-2.2/kg depending on waste | $300/m3 |  |  |
| Catalytic hydrogenation 1 | Hydrodec Group PLC | 5-50ppm PCB:$0.40/L 50-500ppm PCB: $0.80/L >500ppm PCB: $4.00/L |  |  |  |
| Gas Phase Chemical Reduction 1 | Natural Energy Systems Inc. | $2,300/tonne - for 100% PCB waste | $500/tonne - assumes low % of PCB. | $1,300/tonne - assumes 40% PCB. | $1,300/tonne - assumes 40% PCB. |
| Solvated Electron Technology 1 | Oasis Systems /Commodore | $5512 - $6614/tonne | $5512 - $6614/tonne | $5512 - $6614/tonne | $5512 - $6614/tonne |
| Sodium Reduction 1 | Kinectrics Inc | $0.9 - $7/litre |  | $500-1700/tonne | $500-1700/tonne |
| Sodium Reduction 1 | ESI Group | $0.35 to $0.85/litre (up to 3000ppm) | $680 to $1,700/tonne | $1,700 to $4,250/tonne | $800 to $1,220/tonne |

NOTE: Original text of the subsection continues

Economics

The concentration range mentioned in the above Applicability section, is the typical range in which this technique is found to be economically viable. Technically there is not a problem when applied at higher concentrations but it has been found that there are other types of more economically viable PCB treatment processes.

TWG, please confirm the PCB concentration range

From ex-Section 3.3.1 "Waste IN in physico-chemical treatments"

Feed and output stream

Waste IN in the treatment of specific wastes

Hydrogenation of POPs

According to the US DOE, this system can handle most types of waste, including landfill leachates, lagoon bottoms, soils, sludges, liquids, and gases. However, the US DOE has cautioned that reaction by-products and intermediate reduction products need to be monitored in the off-gas from both the reduction process and the boiler. It also noted the need to determine the fate of mercury and other volatile inorganics.

The process is non-discriminatory; that is to say, organic molecules such as PCBs, PAHs, chlorophenols, dioxins, chlorobenzenes, pesticides, herbicides and insecticides are quantitatively converted to methane.

Solvated electron technique

Table 5.203 below shows the input waste IN and output waste OUT for the treatment of chlorinated waste by the solvated electron technique.

| Table 5.203: Products and disposal options for the use of the solvated electron technique Material treated | Products | Disposal options |
| --- | --- | --- |
| Concentrated PCBs | Biphenyl, Ca(OH)2, CaCl2 | Landfill as salts |
| PCBs in soils | Biphenyl, Ca(OH)2, CaCl2, clean soil | Return soil to ground |
| PCBs on surfaces | Biphenyl, Ca(OH)2, CaCl2 | Collect salts and landfill |
| PCB/oil mixtures | Biphenyl, Ca(OH)2, CaCl2, oil | Reuse oil and landfill salts |
| Concentrated HCB | Benzene, NaCl, NaOH | Separate benzene and landfill salts |
| HCB in soils | Benzene, NaCl, NaOH, clean soil | Separate benzene and return soil with salt to ground |
| Source: [ 69, UNEP 2000 ] |  |  |

From ex-Section 4.3.3.1 "Treatment of oil contaminated with PCB"

Achieved environmental benefits

The output of dehalogenation is an oil with dielectric characteristics that make it possible for it to be reused for the same purpose, with a PCB content of less than 1 ppm.

Example plants

One example is operational in Italy.

Reference literature

[ 30, Ecodeco 2002 ] [ 98, WT TWG 2004 ] [146, PCT]

From ex-Section 2.3.5 "Physico-chemical treatments of other wastes"

5.8.1.2 Current emission and consumption levels

From ex-Section 3.3.3.3 "Emissions from the treatment of specific wastes"

Hydrogenation of POPs

At a Canadian facility, product gas was found to contain no detectable PCBs, while concentrations of total chlorobenzenes and dioxins were similar to those of the blank solutions. This suggests that the product gas did not contain quantifiable levels of these substances. When the product gas was burnt in the steam reformer, dioxins were emitted from the reformer stack at a concentration of 15 pg TEQ/Nm3. This dioxin formation was attributed to the use of PCB-contaminated air from the site of the reformer’s combustion air.

All outputs from the destruction of PCB-contaminated waste using this process can be contained and tested. There are no uncontrolled emissions from the process which could result in releases of PCB-contaminated air, solids or liquids to the surrounding environment. Releases of PCBs to the environment could only occur during waste handling prior to treatment, where the possibility of spillage exists.

Because the reaction takes place in a reducing atmosphere in the absence of oxygen, the possibility of dioxin and furan formation is said to be eliminated. If the hydrogen content is maintained at percentages greater than 50 % (dry basis), the formation of PAHs is prevented.

Table 5.205 shows the reported destruction efficiency of hydrogenation processes.

Performance of the processes

| Table 5.205: Reported destruction efficiency of hydrogenation processes Type of waste | Destruction efficiency (%) | Destruction and removal efficiency (DRE) (%) |
| --- | --- | --- |
| PCB oils | 99.999808–99.9999996 | 99.9999985–99.9999997 |
| Chlorobenzenes | 99.9999836–99.9999972 | 99.9999842–99.9999985 |
| Dioxins present in PCB oils | 99.999–99.9999 | NI |
| NI: no information Source: [ 69, UNEP 2000 ] |  |  |

Table 5.206 gives an overview of the emissions to air of Plant 191 which carries out decontamination and dismantling of PCB-containing equipment. VOC emissions are generated by the use of solvent for equipment washing. There is no water release reported for this process.

| Table 5.206: Emissions to air from Plant 191 Parameter/ pollutant measured | Type of measurement | Average | Number of measurements during the three reference years |
| --- | --- | --- | --- |
| Flow (m3/h) | Periodic | 55 825 | 9 |
| NMVOC (mg/Nm3) | Continuous | 74 | NA |
| PCB (μg/Nm3) | Periodic | 0.5 | 8 |
| dl-PCB (ng I-TEQ/Nm3) | Periodic | 0.06 | 9 |
| NA: not applicable |  |  |  |

5.8.1.3 Techniques to consider in the determination of BAT

5.8.1.3.1 Optimising the environmental performance of PCB decontamination

Description

Design and operational measures to prevent and reduce emissions generated by PCB-containing waste decontamination.

Technical description

a) Design measures to prevent dispersion of PCBs from the storage and treatment area:

 dedicated storm and run-off water collection system;

 resin coating applied to the whole concrete floor of the storage and treatment area (since most standard concrete floors absorb PCBs).

b) Implementation of staff access rules to prevent dispersion of contamination:

 accesses to storage and treatment areas are locked;

 special qualification needed to access the area where the waste or equipment polluted by POPs is handled;

 prior to accessing the restricted area, staff put on individual protective outfit depending on the POP in a 'clean' cloakroom;

 after leaving the restricted area, staff go to a 'dirty' cloakroom where the contaminated individual protective outfit is managed with special care.

c) Monitoring of PCB emissions to air.

d) Prevention of liquid PCB dispersion during the decontamination process:

 procedures for filling, emptying and (dis)connecting the vacuum vessel;

 external surfaces of the electrical transformers to be decontaminated are cleaned with anionic liquid; pumping the PCB oil out of the transformer with a pump or under vacuum instead of gravity emptying;

 long period of dripping (12 hours at least) to avoid any PCB drop during further treatment operations, after the separation of the core from the casing.

e) Control of emissions to air:

 the ambient air of the decontamination workshop is treated on activated carbon filters;

 the vacuum pump exhaust is connected to a high-temperature kiln (PCB) or activated carbon filters.

f) Management of waste treatment residues:

 porous parts of the contaminated transformer (wood and paper) are incinerated in a high-temperature kiln (1100 °C) and/or processed in one of non-combustion technologies (e.g. GPCR);

 destruction of the PCBs in the oils (dechlorination, hydrogenation, solvated electron processes).

Achieved environmental benefits

All techniques aim at increasing the environmental performance of PCB decontamination.

Environmental performance and operational data

No information available.

Cross-media effects

 Possible consumption of reagents for the decontamination and/or the abatement system.

 Possible cross-media effects during the incineration of residues.

Technical considerations relevant for applicability

Generally applicable.

Economics

No information available.

Driving force for implementation

 POPs regulation (EC 850/2004).

 Occupational safety

Example plants

In Plant 191, materials containing PCBs are introduced into closed heating vessels under depression (autoclaves) equipped with i) a vacuum pump to suck out air/steam mixtures and ii) a condenser to recover the solvent (prior to further distillation). The opening of the autoclaves is strictly controlled, and performed only after complete cooling. The complete PCB working zone (including dismantling workshop, autoclaves, distillation column, etc.) is located in specific bunded areas, within buildings equipped with devices for airstream collection. The various collected airstreams (process vents/fugitive emissions) are thoroughly treated, by high-temperature incineration and activated carbon adsorption respectively.

The solvent is regenerated through distillation on site, allowing the consumption of such reactants to be minimised, as well as the reduction of the final wastes requiring destruction through high-temperature incineration. VOC emissions from the whole process including solvent loading/regenerating operations are carefully prevented or treated, and monitored.

Reference literature

PCB, santé et environnement, Thierry Meunier, Jean-Claude Amiard, Marc Babut, Expected to be published end 2015 by Lavoisier TEC & DOC

TWG Please provide this article when published.