5.8.1.3.x Optimising the environmental performance of POPs destruction in physico-chemical treatment processes

Description

Design and operational measures to prevent and reduce emissions and releases generated by POPs destruction physico-chemical treatment.

Technical description

- Design measures to prevent dispersion of POPs from the storage and treatment areas

 Dedicated storm and run-off water collection system.

 Resin coating applied to the whole concrete floor of the storage and treatment areas without using POPs in that resin itself.

- Decontamination of soil and other solid wastes which can create dust dispersion

 Mechanical treatment of solid wastes (including soil, debries etc.) contaminated with POPs (materials subjected to decontamination) and the processing units are covered with a "tent" with Negative Air System (NAS).

- Implementation of staff access rules to prevent dispersion of contamination

 Accesses to storage and treatment areas are locked.

 Special qualification is required to access the area where the waste or polluted equipment is handled.

 Separate 'clean' and 'dirty' cloakrooms to put on/take off individual protective outfit.

- Control of emissions to air from NAS

 The NAS gas outlets contain filters (using most appropriate method in order to prevent POPs releases and/or formation), and verifying measurements are carried out of the released POPs and other relevant pollutants including those contaminating soil and unintentional POPs (PCDD/Fs, dioxin-like PCBs, hexachlorobenzene).

- Management of waste treatment residues

 Residues are treated in physico-chemical treatment technologies for POPs destruction.

- Management of the proces with regards to unintentional POPs formation

 Technology runs within conditions which do not lead to unintentional POPs (e.g. PCDD/Fs) formation when they are described for certain technology

- Management of treated waste (soil in particular) when it doesn’t reach the target concentration of POPs in treated material

 Treated material is re-processed in technology when it doesn’t meet required target concentration of POPs set in permit which should not be higher than standards set for the purpose of material use (e.g. standards set for industrial areas national legislation etc.)

- Monitoring of outlets

 All outlets are controlled for their content of POP contaminant(s) treated in the waste (e.g. DDT and other relevant OCPs in case of contaminated material from obsolete pesticides storage), as well as for unintentional POPs content (PCDD/Fs as minimum requirement). These measurements should be carried out in order to evaluate destruction efficiency (DE) and destruction and removal efficience (DRE) of applied technology

Achieved environmental benefits

All techniques aim at increasing the environmental performance of destruction and irreversible transformation of POPs in waste.

Environmental performance and operational data

Destruciton efficiencies (DE) well above 99.999% for all treated POPs. Air emissions of PCDD/Fs below 0.05 ng TEQ/Nm3.

Cross-media effects

 Possible consumption of reagents for the decontamination and/or the abatement system.

 Possible cross-media effects during the incineration of residues.

Technical considerations relevant for applicability

Generally applicable.

Economics

No information available.

Driving force for implementation

 POPs regulation (EC 850/2004).

 Occupational safety

Example plants

Brief description of the technology used for decontamination of buildings and soil contaminated with PCDD/Fs and OCPs

The technology used for decontamination of the contaminated site consisted, essentially, of two steps.

The first step includes thermal desorption, it means indirectly heated rotary kiln (ITD), heated by gas burners. The desorbate is moved in an inert gas stream to an air pollution control system (APS). Further, the separated organic portion is led to the actual BCD unit.

In the case of metal waste, electrically heated annealing furnace (MPF) is used for decontamination. Its gas products are led to the APS unit, where organic portion is separated, and subsequently lead to the BCD unit.

The second step is the actual BCD technology, serving for decomposition of the concentrated organic portion. The technological BCD section is still supplemented by electrically heated pressure reactor (3.9 MPa) serving for decomposition of the organic condensate from the APS. This unit is very important, because it may be used, when necessary, for decomposition of essentially any intermediates (for example, in the case of possible presence of high-molecular BCD products in the process oil).

The technology line is supplemented by the above mentioned APS separation unit, for capture and separation of intermediates from the ITD, MPF, as well as BCD equipment. This unit contains, at the gas outlet, textile bag filters and filters with activated carbon.

Catalytic dechlorination in alkaline environment BCD (Base Catalysed Dechlorination) was developed by USEPA in 1989. It is based on thermal destruction of halogenated aliphatic and aromatic organic substances by means of compounds of alkali metals and/or caustic alkali metals (in this case, NaOH). The contaminated materials may be present either in water or organic environment, wherein the destruction process proceeds only after de-watering of the reaction system. The destruction process proceeds on condition that a hydrogen donor (for example, industrial liquid paraffin), catalyst, and carbon source, are present in the system. The given reaction time is in the range 0.5 - 3 hours, at temperatures 200 - 400 °C in the pressure reactor. The pressure destruction of POPs proceeds also in the presence of calcium hydroxide at 100 - 300 °C. The specific technology used utilized the temperature of 350 °C, sodium hydroxide, and mineral oil SUNPAR 107 H. 1-Hexadecanol is used as the catalyst. The likely detoxification mechanism, in the given conditions, is radical dehalogenation, possibly also nucleophilic substitution of halogen by hydrogen.

The actual buildings subjected to the decontamination, and the processing units, were covered with a "tent" with Negative Air System (NAS). The system consisted of 4 lines with the total air exchange output of 210 m3 per hour. The NAS gas outlets contained filters, and verifying measurements were carried out of the released pollutants (see Table No. xxx).

In total, over 40 thousand tons of soils and materials from building demolition were subjected to the decontamination process. The actual BCD reactor processed 150 tons of material, because it was preceded by indirect thermal desorption (ITD) and annealing furnace (MPF). The actual BCD reactor processed a process oil, where also POPs were concentrated. Information on inputs and outputs, i.e., POPs concentrations in oil after processing in the reactor, is available only from the pilot operation of the unit .

For full scal facility in the Czech Republic: In the mixed waste water samples, monitored in the interval of one to two weeks, there were found PCDD/Fs concentrations between 0 and 1.4 ng TEQ/l (average 0.19; median 0.1; standard deviation 0.28). In the mixed samples of soil after decontamination, the PCDD/Fs concentrations were from 0 to 3.3 ng TEQ/g (average 0.07; median 0.01; standard deviation 0.26) and OCPs concentrations from < 0.1 to 7.9 mg/kg (average 0.81; median 0.41; standard deviation 1.02). Soils with concentrations exceeding the target decontamination limits of 2004 (1 ng TEQ/g PCDD/Fs, and 30 mg/kg OCPs) were returned back to repeated decontamination .

Levels of OCPs in waste inputs as well as in output material are shown in Table 5.xxI. The same for PCDD/Fs is summarized in Table 5.xxII.

Table 5.xxI: Hexachlorobenzene and lindane decomposition by the BCD process during pilot operation in Spolana Neratovice. Source .

| Material | Inlet mg/kg |  | Outlet Oil Matrix mg/kg |  |
| --- | --- | --- | --- | --- |
|  | HCB | Lindane | HCB | Lindane |
| Chemical waste | 29,000 | 1,500 | < 1.0 | < 1.0 |
| Chemical waste | 200,000 | 900 | < 2.0 | < 2.0 |
| Chemical waste | 550,000 | 1,000 | < 2.0 | < 2.0 |
| Chemical waste | 270,000 | 1,000 | < 2.0 | < 2.0 |
| Chemical waste | 160,000 | 1,000 | < 2.0 | < 2.0 |
| Dust | 7,600 | 7 | < 2.0 | < 2.0 |
| Chemical waste | 1,598 | 19,000 | < 2.0 | < 2.0 |
| Concentrate Aqueous | 630 | < 2.0 | < 2.0 | < 2.0 |
| Concentrate Organic | 11,000 | < 2.0 | < 2.0 | < 2.0 |

Table 5.xxII: Dioxin concentrations at inputs and outputs of the ITD and BCD technologies, according to the study published in Poland. Source .

| Levels of PCDD/PCDF [TEQ ng/g] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix | Technology |  |  |  |
|  | ITD/BCD |  | BCD |  |
|  | inlet | outlet | inlet | outlet (oil) |
| concrete | 4780 | 0.066 |  |  |
| soil | 45 | 0.0030 |  |  |
| concentrate aqueous from ITD |  |  | 96170 | 0 |
| waste chemicals |  |  | 209 | 0 |

References:

BCD (2008a). Projekt Spolana - dioxiny. Roční zpráva o plnění podmínek rozhodnutí KÚSK 37676/17168/2005/OŽP. Neratovice, BCD CZ, a.s.: 33.

Kraitr, M., V. Richtr and V. Sirotek (2006). "Unieszkodliwanie dioksyn (PCDDs/PCDFs) ze starych obciąźeń środowiska za pomocą technologii zasadowego rozkładu katalitycznego (BCD). Disposal of Polychlorinated Dioxin Compounds (PCDDs/PCDFs) from Old Environmental Load by the Base Catalysed Decomposition (BCD) Technology." Chemia - dydaktyka - ekologia - metrologia 11(1-2): 35-41.

Kubal, M., J. Fairweather, P. Crain and M. Kuraš (2004). Treatment of solid waste polluted by polychlorinated contaminants (pilot-scale demonstration). International Conference on Waste Management and the Environment No2. S. WIT Press, ROYAUME-UNI  (2004) (Monographie). Rhodes, WIT Press: 13-23.